| Description | Times | Actual Data |
| --- | --- | --- |
| Prepare the reactor: | Start: __:__ End: __:__ |  |
| Load required amount of Methanol into Reactor: 1. Weigh the required amount of Methanol using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose TP.3 ATI. 3. Using the peristaltic pump, load Methanol via the loading port. 4. Set/Keep stirring rate in reactor 002-XX to 150-200rpm. 5. Set temperature on thermostat 011-XX to 15-25°C. 6. Keep the temperature of reaction mixture in range: 15-25°C. 7. Addition is done - in one portioin. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual Set temp: ......... °C; Actual Temp. of rm: ......... °C; Actual Stirring: ......... rpm; Actual Addition rate: .........; |
| Load required amount of IP.2 into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of IP.2 using balance 007-XX into a jug TP.3 ATI. 3. Load IP.2 into the Reactor via handhole using a plastic scoop TP.3 ATI. 4. Set/Keep stirring rate in reactor 002-XX to 150-200rpm. 5. Set temperature on thermostat 011-XX to 15-25°C. 6. Keep the temperature of reaction mixture in range: 15-25°C. 7. Addition is done - in one portioin. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual Set temp: ......... °C; Actual Temp. of rm: ......... °C; Actual Stirring: ......... rpm; Actual Addition rate: .........; |
| Load required amount of Hydrochloric acid 35% into Reactor: 1. Weigh the required amount of Hydrochloric acid 35% using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose TP.3 ATI. 3. Using the peristaltic pump, load Hydrochloric acid 35% via the loading port. 4. Set/Keep stirring rate in reactor 002-XX to 150-200rpm. 5. Set temperature on thermostat 011-XX to 15-25°C. 6. Keep the temperature of reaction mixture in range: 15-25°C. 7. Addition is done - in one portioin, pump with perist pump (60-80%). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual Set temp: ......... °C; Actual Temp. of rm: ......... °C; Actual Stirring: ......... rpm; Actual Addition rate: .........; |
| Start a heating/cooling operation: 1. Open cooling water for heating/cooling circulator 011-XX 2. Set temperature on thermostat 011-XX to 75-80°C. 3. Keep the temperature of reaction mixture in range: 65-69°C (mild reflux). 4. Provide temperature gradient: straight heating to the required temperature.. 5. Set/Keep stirring rate to 150-200rpm. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual Target Temp.: .........; Actual cooling/heating grad.: .........; Actual Stirring: ......... rpm; |
| Start a hold time operation: 1. Set temperature on thermostat 011-XX to 75-80°C. 2. Keep the temperature of reaction mixture in range: 67-70oC. 3. Adjust stirring rate to 150-200rpm. 4. Hold for duration: 3-6h (max 6h with IPC waiting time).. 5. Optional: Possible pause - NO. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual Target Temp.: .........; Actual Stirring: ......... rpm; Actual Time: .........; Actual break point: .........; |
| In-process control: 1. Expected Result: conv. >99%. 2. On IPC failure: continue heating for around 1-2h more and repeat IPC. 3. Estimated time for analysis: NLT 3h. | Start: __:__ End: __:__ | Actual Expected Result: .........; Actual IPC Failure Action: .........; Actual Estim. Time of analysis: .........; |
| Start a heating/cooling operation: 1. Open cooling water for heating/cooling circulator 011-XX 2. Set temperature on thermostat 011-XX to 0->15°C. 3. Keep the temperature of reaction mixture in range: 15-25°C. 4. Provide temperature gradient: straight cooling to the required temperature.. 5. Set/Keep stirring rate to 150-200rpm. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual Target Temp.: .........; Actual cooling/heating grad.: .........; Actual Stirring: ......... rpm; Actual break point: .........; |
| Load required amount of Sodium hydroxide into the dropping funnel of Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of Sodium hydroxide using balance 007-XX into a jug TP.3 ATI. 3. Load Sodium hydroxide into the dropping funnel using a plastic funnel TP.3 ATI. 4. Set/Keep stirring rate in reactor 002-XX to 150-200rpm. 5. Adjust pH to: 8-9 [WARNING, PARAMETER IS PC]. 6. Set temperature on thermostat 011-XX to -10…-15°C. 7. Keep the temperature of reaction mixture in range: 0-30°C. 8. Addition is done - addition is temperature controlled. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual Set temp: ......... °C; Actual Temp. of rm: ......... °C; Actual Stirring: ......... rpm; Actual Addition rate: .........; Actual pH check: .........; |
| Start a heating/cooling operation: 1. Open cooling water for heating/cooling circulator 011-XX 2. Set temperature on thermostat 011-XX to -5…0°C. 3. Keep the temperature of reaction mixture in range: 10-15°C. 4. Provide temperature gradient: straight cooling to the required temperature.. 5. Set/Keep stirring rate to 150-200rpm. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual Target Temp.: .........; Actual cooling/heating grad.: .........; Actual Stirring: ......... rpm; Actual break point: .........; |
| Start a hold time operation: 1. Set temperature on thermostat 011-XX to 5…15°C. 2. Keep the temperature of reaction mixture in range: 10-15°C. 3. Adjust stirring rate to 150-200. 4. Hold for duration: 40-50min. 5. Optional: Possible pause - YES, when pH is 8-9. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual Target Temp.: .........; Actual Stirring: ......... rpm; Actual Time: .........; Actual break point: .........; |
| Filter 046-XX setup: 1. Place a PTFE seal on filtrate receiver top edge. 2. Connect suspension receiver to filtrate receiver 3. Connect hose TP.3 ATI to filtrate receiver vacuum valve. Connect other end of hose to a membrane pump 001-XX. Membrane pump 001-XX connected to receiver vacuum valve:  Filtration cloth is cut and installed properly:  | Start: __:__ End: __:__ |  |
| Filter the material: 1. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 2. Continue until: when Filter Cake is dry.. Use a spoon TP.3 ATI to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual End point: .........; Actual vacuum: .........; |
| Wash filter cake: 1. Stop membrane pump 001-XX. 2. Use required amount of Water for washing Filter cake 3. Ensure loaded material temperature is rt/not controlled. 4. Mix on filter: manual on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual Loaded material temp.: .........; Actual Mixing on filter: .........; |
| Filter the material: 1. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 2. Continue until: when Filter Cake is dry.. Use a spoon TP.3 ATI to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual End point: .........; Actual vacuum: .........; |
| Wash filter cake: 1. Stop membrane pump 001-XX. 2. Use required amount of Water for washing Filter cake 3. Ensure loaded material temperature is rt/not controlled. 4. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual Loaded material temp.: .........; Actual Mixing: .........; |
| Filter the material: 1. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 2. Continue until: when Filter Cake is dry.. Use a spoon TP.3 ATI to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual End point: .........; Actual vacuum: .........; |
| Dry the filter cake on filter under vacuum: 1. After all material have been transferred on filter, maintain it on the filter under vacuum. 2. Consider air, moisture, and light sensitivity of the material. 3. After finishing drying on filter, pump the filtrate from the filter into waste canister and weigh it. | Start: __:__ End: __:__ | Actual Time: .........; Actual vacuum: .........; |
| Wash filter cake: 1. Stop membrane pump 001-XX. 2. Use required amount of 2-propanol for washing Filter cake 3. Ensure loaded material temperature is not controlled. 4. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual Loaded material temp.: .........; Actual Stirring: ......... rpm; |
| Start a hold time operation: 1. Set temperature on thermostat 011-XX to tbd. 2. Keep the temperature of reaction mixture in range: 0-4°C [WARNING, PARAMETER IS CY]. 3. Adjust stirring rate to 100-120Torr. 4. Hold for duration: 20-30min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual Target Temp.: .........; Actual Stirring: ......... rpm; Actual Time: .........; |
| Filter the material: 1. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 2. Continue until: when FK is dry.. Use a spoon TP.3 ATI to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual End point: .........; Actual vacuum: .........; |
| Load required amount of 2-propanol into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of 2-propanol using balance 007-XX into a jug TP.3 ATI. 3. Pour 2-propanol into the Reactor via handhole using a plastic funnel TP.3 ATI. 4. Addition is done - addition in one portion. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual Addition rate: .........; |
| Filter the material: 1. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 2. Continue until: when FK is dry.. Use a spoon TP.3 ATI to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual End point: .........; Actual vacuum: .........; |
| Dry the filter cake on filter under vacuum: 1. After all material have been transferred on filter, maintain it on the filter under vacuum. 2. Consider air, moisture, and light sensitivity of the material. 3. After finishing drying on filter, pump the filtrate from the filter into waste canister and weigh it. | Start: __:__ End: __:__ | Actual Time: .........; Actual vacuum: .........; |
| Dry the product in vacuum oven 012-XX: 1. Check and record the tare mass of each tray into table XX.1. 2. Transfer the product (filter cake) onto trays with a layer thickness of 4cm. 3. Place the trays in the vacuum oven. 4. Connect vacuum pump to the oven via cold trap with dry ice. 5. Set the oven temperature to; 40°C and start the pump with set: . 6. Dry the product, checking the weight periodically (NLT 12h after start of drying) to determine dryness as long as needed.. 7. Perform Mix/delump: yes. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual Vacuum: .........; Actual layer thickness: .........; Actual mixing/delumping: .........; Actual Time: .........; Actual Exp. Yield: .........; Actual Exp. Mass: .........; |
| Finish drying: 1. Save heating/cooling circulator 011-XX temperature logs on the USB stick that is inside the control panel or in the USB ports. 2. Turn off membrane pump and repressurize the oven. Turn off the oven. 3. Remove the trays from the oven and weigh them. Calculate loss on drying and fill Table X. 4. Calculate process yield: | Start: __:__ End: __:__ |  |
| after drying prod. Is sampled and packed. | Start: __:__ End: __:__ | Actual pack. Mat.: .........; Actual Analyt sample: .........; Actual Ret sample: .........; |
| Perform the analysis according to QDQS: 1. If analysis result is YES, proceed accordingly. 2. If analysis result is NO, repeat necessary steps. | Start: __:__ End: __:__ | Actual YES: .........; Actual NO: .........; |
| Reactors and filters are cleaned | Start: __:__ End: __:__ | Actual solvent: .........; Actual time required: .........; |
| Perform waste treatment: | Start: __:__ End: __:__ |  |