| OP. NUMBER | Description | Times | Actual Data |
| --- | --- | --- | --- |
| 1 | Reactor 002-XX preparation: 1. Heat Reactor to 120oC. 2. Hold Reactor at temperature for 10-15min. 3. Set Argon flow during heating to 5-10 L/min. 4. Cool Reactor to 15-25oC. 5. Set Argon flow during cooling to 1-2 L/min. COMMENT INFO: visually check the condenser for condensate before drying | Start: __:__ End: __:__ |  |
| 2 | Load DCM into reactor 002-XX: 1. Weigh DCM on balance 007-XX 2. Set/Adjust stirring rate in reactor to 150-200rpm (range is recommended and can be adjusted). 3. Set/Adjust inert gas flow rate to 1-2 L/min. 4. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 5. Keep reaction mixture temperature in range: 15-25oC. 6. Addition speed - in one portion. 7. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 3 | Load Imidazole carboxaldehyde (DETO SM.2) into reactor 002-XX: 1. Remove secondary package and open primary package. 2. Weigh Imidazole carboxaldehyde (DETO SM.2) on balance 007-XX into jug "TP.1 DETO". 3. Load Imidazole carboxaldehyde (DETO SM.2) into reactor via handhole using a plastic scoop and plastic funnel "TP.1 DETO". 4. Set/Adjust stirring rate in reactor to 150-200rpm (range is recommended and can be adjusted). 5. Set/Adjust inert gas flow rate to 1-2 L/min. 6. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 7. Keep reaction mixture temperature in range: 15-25oC. 8. Addition speed - in portions. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) COMMENT INFO: dusty solid, consider loading in 1kg portions. | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 4 | Load DMSA-Cl into reactor 002-XX: 1. Weigh DMSA-Cl on balance 007-XX 2. Set/Adjust stirring rate in reactor to 150-200rpm (range is recommended and can be adjusted). 3. Set/Adjust inert gas flow rate to 2-5 L/min. 4. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 5. Keep reaction mixture temperature in range: +20… +30oC. 6. Addition speed - pump with perist pump, temp controlled. 7. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) COMMENT INFO: liquid. Keep condenser running and control temp. it is toxic, corrosive wear mask and protection. Keep below 30oC. consider filtration it via p1-2 filter prior to loading??? Use p1 filter to filter it into 10L canister before loading into reactor in 8kg tech batch took 15min | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 5 | Load DCM into reactor 002-XX: 1. Remove secondary package and open primary package. 2. Weigh DCM on balance 007-XX into jug "TP.1 DETO". 3. Pour DCM into reactor via handhole using a plastic funnel "TP.1 DETO". 4. Set/Adjust stirring rate in reactor to 150-200 (range is recommended and can be adjusted). 5. Set/Adjust inert gas flow rate to 2-5 L/min. 6. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 7. Keep reaction mixture temperature in range: +20… +30oC. 8. Addition speed - one portion. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) COMMENT INFO: Rinse the canister from previous op too. | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 6 | Start a hold time: 1. Set/Adjust heating/cooling circulator 011-XX to 20oC. 2. Keep reaction mixture temperature in range: 20-30oC for 50-60min. 3. Set/Adjust stirring in reactor 002-XX to 150-200rpm (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to 2-5 L/min. Fill Table X at least every Y minutes. COMMENT INFO: in order to mix properly. Reagents already start to react with each other slowly. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 7 | Load Triethylamine into reactor 002-XX: 1. Weigh Triethylamine on balance 007-XX 2. Transfer Triethylamine into reactor with peristaltic pump and hose "TP.1 DETO". Set peristaltic pump to 10-12%;. 3. Set/Adjust stirring rate in reactor to 150-200rpm (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to 1-2 L/min. 5. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 6. Keep reaction mixture temperature in range: +20… +30oC [WARNING, PARAMETER IS PC]. 7. Addition speed - pump with perist pump. Portionwise. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) COMMENT INFO: no reaction at this step. check numbers in pgi Keep below 30oC. First portions are added carefully. Perist pump setting should be slow, like 10-15%. Smells | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual peristaltic pump set: ........ %; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 8 | Load DCM into reactor 002-XX: 1. Remove secondary package and open primary package. 2. Weigh DCM on balance 007-XX into jug "TP.1 DETO". 3. Pour DCM into reactor via handhole using a plastic funnel "TP.1 DETO". 4. Set/Adjust stirring rate in reactor to 150-200rpm (range is recommended and can be adjusted). 5. Set/Adjust inert gas flow rate to 1-2 L/min. 6. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 7. Keep reaction mixture temperature in range: 20-30oC. 8. Addition speed - in one portion. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 9 | Start a hold time: 1. Set/Adjust heating/cooling circulator 011-XX to 20oC. 2. Keep reaction mixture temperature in range: 20-30oC for 15-25min. 3. Set/Adjust stirring in reactor 002-XX to 150-200rpm (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to 1-2 L/min. Fill Table X at least every Y minutes. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 10 | Start heating/cooling with heating/cooling circulator 011-XX: 1. Check that cooling water for 011-XX is opened. 2. Set/Adjust temperature on 011-XX to 20-35oC (keep jacket/RM temp. difference below 50oC). 3. Keep reaction mixture temperature in range: +20… +30oC; [WARNING, PARAMETER IS PC]. 4. Set/Adjust stirring in reactor 002-XX to 150-200rpm (range is recommended and can be adjusted). 5. Set/Adjust inert gas flow rate to 2-5 L/min. Fill Table X at least every Y minutes. COMMENT INFO: still a chanse of runaway reaciton. Straight | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 11 | Start a hold time: 1. Set/Adjust heating/cooling circulator 011-XX to 28oC. 2. Keep reaction mixture temperature in range: +20… +30oC; [WARNING, PARAMETER IS PC] for 12-44h. 3. Set/Adjust stirring in reactor 002-XX to 150-200 (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to 1-2 L/min. 5. Optional: Possible pause - keep overnight. Fill Table X at least every Y minutes. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 12 | In-process control: In-process control (determination of IP.X conversion after 12-41h): 1. Take approximately 1mL of reaction mixture into a 20 mL vial; 2. Purge the vial with argon; 3. Label the sample with an IPC number. For example: TBD-0XXX-Y IP.? RM IPC X, where Y is the batch number (copied from the header) and X is the IPC serial number. 4. Submit the sample to QC for analysis. 5. Keep stirring the reaction mixture until the IPC result is available. Decision criteria based on IPC result: • If IP.X conversion is conv. NLT95% [WARNING, PARAMETER IS CY], proceed with the workup (Operation XX). • On IPC failure: more time, 4-6h more, in extreme case add more dmsa-cl (10% from initial loading).. IPC 1: TBD-0XXX-Y IP.X RM IPC 1 Results: Corresponds to RelS requirements:  - Yes  - No IP.X conversion: ..... % COMMENT INFO: in pre-val check IPC showed: 17h - 10% / 86%; 21h - 8.8% / 91%; 41h - 6% / 93%; if conv. Lower than 80%. Increase temp to 35-40oC | Start: __:__ End: __:__ | Result: ..............; |
| 13 | Load water into reactor 002-XX: 1. Weigh water on balance 007-XX 2. Set/Adjust stirring rate in reactor to 150-200 (range is recommended and can be adjusted). 3. Set/Adjust temperature on heating/cooling circulator 011-XX to 10oC (keep jacket/RM temp. difference below 50oC). 4. Keep reaction mixture temperature in range: +10… +30oC;. 5. Addition speed - temp. controlled, pump with perist pump with portions. 6. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) COMMENT INFO: some exothermic effect in 5.9kg scale addition took 5 min in 8kg scale addition took 1h | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 14 | Start a hold time: 1. Set/Adjust heating/cooling circulator 011-XX to 20oC;. 2. Keep reaction mixture temperature in range: +10… +30oC; for 30-40min. 3. Set/Adjust stirring in reactor 002-XX to 150-200 (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to OFF. Fill Table X at least every Y minutes. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 15 | Perform extraction/separation: 1. Set/Adjust stirring in reactor to OFF (recommended range) (range is recommended and can be adjusted). 2. Stop stirring and let layers completely separate (visual check). Expected time of separation 10-15min. 3. Perform phase separation. Bottom "description" phase is stored into canister "TP.1 DETO", top "description" layer is stored in canister "TP.1 DETO"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) COMMENT INFO: product goes into dcm. difficult to see phase separation, use flashlight to check | Start: __:__ End: __:__ | Actual stirring set: ........ rpm; Actual separation time: ........ min; |
| 16 | Load water into reactor 002-XX: 1. Weigh water on balance 007-XX 2. Set/Adjust stirring rate in reactor to 150-200 (range is recommended and can be adjusted). 3. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 4. Keep reaction mixture temperature in range: +10… +30oC;. 5. Addition speed - one portion. Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 17 | Start a hold time: 1. Set/Adjust heating/cooling circulator 011-XX to 20oC. 2. Keep reaction mixture temperature in range: +10… +30oC; for 5-10min. 3. Set/Adjust stirring in reactor 002-XX to 150-200 (range is recommended and can be adjusted). Fill Table X at least every Y minutes. COMMENT INFO: consider separation water from the top of reactor. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 18 | Perform extraction/separation: 1. Set/Adjust stirring in reactor to OFF (recommended range) (range is recommended and can be adjusted). 2. Mix layers for al long as needed. 3. Perform phase separation. Bottom "description" phase is stored into canister "TP.1 DETO", top "description" layer is stored in canister "TP.1 DETO"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) | Start: __:__ End: __:__ | Actual stirring set: ........ rpm; |
| 19 | Load water into reactor 002-XX: 1. Weigh water on balance 007-XX 2. Set/Adjust stirring rate in reactor to 150-200 (range is recommended and can be adjusted). 3. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 4. Keep reaction mixture temperature in range: +10… +30oC;. 5. Addition speed - one portion. Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 20 | Start a hold time: 1. Set/Adjust heating/cooling circulator 011-XX to 20oC. 2. Keep reaction mixture temperature in range: +10… +30oC; for 5-10min. 3. Set/Adjust stirring in reactor 002-XX to 150-200 (range is recommended and can be adjusted). Fill Table X at least every Y minutes. COMMENT INFO: consider separation water from the top of reactor. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 21 | Perform extraction/separation: 1. Set/Adjust stirring in reactor to OFF (recommended range) (range is recommended and can be adjusted). 2. Mix layers for al long as needed. 3. Perform phase separation. Bottom "description" phase is stored into canister "TP.1 DETO", top "description" layer is stored in canister "TP.1 DETO"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) | Start: __:__ End: __:__ | Actual stirring set: ........ rpm; |
| 22 | Load water into reactor 002-XX: 1. Weigh water on balance 007-XX 2. Set/Adjust stirring rate in reactor to 150-200 (range is recommended and can be adjusted). 3. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 4. Keep reaction mixture temperature in range: +10… +30oC;. 5. Addition speed - one portion. Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 23 | Start a hold time: 1. Set/Adjust heating/cooling circulator 011-XX to 20oC. 2. Keep reaction mixture temperature in range: +10… +30oC; for 5-10min. 3. Set/Adjust stirring in reactor 002-XX to 150-200 (range is recommended and can be adjusted). Fill Table X at least every Y minutes. COMMENT INFO: consider separation water from the top of reactor. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 24 | Perform extraction/separation: 1. Set/Adjust stirring in reactor to OFF (recommended range) (range is recommended and can be adjusted). 2. Mix layers for al long as needed. 3. Perform phase separation. Bottom "description" phase is stored into canister "TP.1 DETO", top "description" layer is stored in canister "TP.1 DETO"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) | Start: __:__ End: __:__ | Actual stirring set: ........ rpm; |
| 25 | Start evaporation in reactor 002-XX: 1. Set/Adjust temperature heating/cooling circulator 011-XX to 60oC with target 20-47oC (keep jacket/RM temp. difference below 50oC). 2. Connect a membrane pump via a cold trap and turn it ON. 3. Gradually reduce pressure 300-400Torr. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 4. Set/Adjust stirring to 150-200rpm (range is recommended and can be adjusted). 5. Continue until: remove NLT40-45% of theoretical DCM. Fill Table X at least every Y minutes. COMMENT INFO: in 5.9kg took 2h | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual pressure: ........ Torr; Actual stirring set: ........ rpm; |
| 26 | Load n-Heptane into reactor 002-XX: 1. Weigh n-Heptane on balance 007-XX 2. Set/Adjust stirring rate in reactor to 150-200 (range is recommended and can be adjusted). 3. Set/Adjust temperature on heating/cooling circulator 011-XX to 40oC (keep jacket/RM temp. difference below 50oC). 4. Keep reaction mixture temperature in range: 20-47oC. 5. Addition speed - in 3 portions with 2-3min interval.. Fill Table X at least every Y minutes. Specified loading: ….. kg (… - … kg) COMMENT INFO: in 3 portions with 2-3min interval. Record observations so walls are not so hot, wait until it is lowered | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 27 | Start evaporation in reactor 002-XX: 1. Set/Adjust temperature heating/cooling circulator 011-XX to 60oC with target 20-47oC (keep jacket/RM temp. difference below 50oC). 2. Connect a membrane pump via a cold trap and turn it ON. 3. Gradually reduce pressure 300-400Torr. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 4. Set/Adjust stirring to 150-200rpm (range is recommended and can be adjusted). 5. Continue until: remove NLT20-25% of theoretical DCM. Fill Table X at least every Y minutes. COMMENT INFO: in 5.9kg took 2h | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual pressure: ........ Torr; Actual stirring set: ........ rpm; |
| 28 | Start heating/cooling with heating/cooling circulator 011-XX: 1. Check that cooling water for 011-XX is opened. 2. Set/Adjust temperature on 011-XX to -10...0-3oC (keep jacket/RM temp. difference below 50oC). 3. Keep reaction mixture temperature in range: 0 … +5oC. 4. Set/Adjust stirring in reactor 002-XX to 100-130rpm (range is recommended and can be adjusted). Fill Table X at least every Y minutes. COMMENT INFO: straight visually check that it is sufficient to move all product | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 29 | Start a hold time: 1. Set/Adjust heating/cooling circulator 011-XX to 1oC. 2. Keep reaction mixture temperature in range: 0 … +5oC for NLT 30min. 3. Set/Adjust stirring in reactor 002-XX to 100rpm (range is recommended and can be adjusted). 4. Optional: Possible pause - keep overnight. Fill Table X at least every Y minutes. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 30 | Filter 046-6-7 setup: 1. Place a PTFE seal on filtrate receiver top edge. 2. Connect suspension receiver to filtrate receiver 3. Install filter cloth into filter and fix it with the plastic ring. 4. Connect hose "TP.1 DETO" to filtrate receiver vacuum valve. Connect other end of hose to a membrane pump 001-XX. Membrane pump 001-XX connected to receiver vacuum valve:  Filtration cloth is cut and installed properly:  Filter 046-2-13 setup: 5. Connect suspension receiver to glass receiver from reactor 002-XX with norprene hose "TP.1 DETO". 6. Connect reactor receiver to membrane pump 001-XX with norprene hose "vacuum". 7. Install filter cloth into filter and fix it with the plastic ring. Filter is connected to Reactor's receiver vessel:  Membrane pump 001-XX is connected to Reactor's receiver vacuum valve:  Filtration cloth is cut and installed properly:  | Start: __:__ End: __:__ |  |
| 31 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.1 DETO" from reactor valve and through peristaltic pump 001-XX to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Set the vacuum pump to 250-350Torr and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Continue until: until Filter cake is dry.. 6. Use a spoon "TP.1 DETO" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "TP.1 DETO". Resume the process afterwards. COMMENT INFO: sefar cloth. Use support not air moisture light sensitive. It smells. Precip it pretty heavy. Unloading from reactor requred manual draining… | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 32 | Wash filter cake with n-Heptane: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use n-Heptane for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: should be very effeciently stirring on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (… - … kg) COMMENT INFO: this can be used to rinse reactor walls and shaft | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 33 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.1 DETO" from reactor valve and through peristaltic pump 001-XX to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Set the vacuum pump to 250-350Torr and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Continue until: until Filter cake is dry.. 6. Use a spoon "TP.1 DETO" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "TP.1 DETO". Resume the process afterwards. COMMENT INFO: sefar cloth. Use support not air moisture light sensitive. It smells. Precip it pretty heavy. Unloading from reactor requred manual draining… | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 34 | Wash filter cake with n-Heptane: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use n-Heptane for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: should be very effeciently stirring on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (… - … kg) COMMENT INFO: this can be used to rinse reactor walls and shaft | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 35 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.1 DETO" from reactor valve and through peristaltic pump 001-XX to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Set the vacuum pump to 250-350Torr and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Continue until: until Filter cake is dry.. 6. Use a spoon "TP.1 DETO" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "TP.1 DETO". Resume the process afterwards. COMMENT INFO: sefar cloth. Use support not air moisture light sensitive. It smells. Precip it pretty heavy. Unloading from reactor requred manual draining… | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 36 | Dry the filter cake on filter under vacuum: 1. Once Filter cake is visually dry, maintain it on the filter under vacuum. 2. Continue to apply vacuum 250-350Torr for 30-50min to remove residual solvents. 3. Use a spoon "TP.1 DETO" to press and remove cracks that might appear on filter cake. 4. Consider air, moisture, and light sensitivity of the material. 5. After finishing drying on filter, pump the filtrate from the filter into waste canister and weigh it. m tara = ....... kg ; m gross weight = ....... kg ; m net "type of waste" waste = ....... kg ; | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 37 | Dry the product in vacuum oven 012-XX: 1. Check and record the tare mass of each tray into table XX.1. 2. Transfer the product (filter cake) onto trays with a layer thickness of NMT 4cm. Divide all wet product approximately evenly between all trays. Crush big lumps with shovel. 3. Place the trays in the vacuum oven. 4. Connect vacuum pump to the oven via cold trap with dry ice. 5. Set overheating protection of the oven to: 60oC. 6. Set the oven temperature to; 40-50oC and start the pump with set: 30Torr. 7. Dry the product, checking the weight periodically (NLT 12h after start of drying) to determine dryness 24-36h. In 2-3h weight loss should be less than 2-3%. 8. Perform Mix/delump: Yes, after at least 12h of starting of drying. Record weights in Table X. Table X filled:  Overheating set: .......°C Actual pressure after at least 1h from the start: ...... Torr Temperature after at least 1h from the start: .......°C COMMENT INFO: lead pipe into ventilation so in validation check all range. in 5.9kg scale took 24h specify that we can use here vacuum oven. It was done in pre-val batches and is sutable for this process too. weight checking of material on trays. In 2-3h weight loss should be less than 2-3% Final decission - to be done in vacuum oven | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual pressure: ........ Torr; |
| 38 | Finish drying: 1. Save heating/cooling circulator 011-XX temperature logs on the USB stick that is inside the control panel or in the USB ports. 2. Turn off membrane pump and repressurize the oven. Turn off the oven. 3. Remove the trays from the oven and weigh them. Calculate loss on drying and fill Table X. 4. Calculate process yield: | Start: __:__ End: __:__ |  |
| 39 | Pack the product: 1. Use analytical sample amount: 2g. 2. Retain sample amount: 4g. 3. Pack using: double PE bag.. 4. Consider air, moisture, and light sensitivity of the material: NO. | Start: __:__ End: __:__ |  |
| 40 | Perform the analysis according to QDQS: 1. If analysis result is YES, proceed accordingly. 2. If analysis result is NO, repeat necessary steps. COMMENT INFO: continue rnd will check. We need re-crystallization? | Start: __:__ End: __:__ |  |
| 41 | Clean the equipment: 1. Follow the solvent sequence: MeOH, Water, Acetone. 2. Estimated time required: 3-5h. | Start: __:__ End: __:__ |  |
| 42 | Perform waste treatment: | Start: __:__ End: __:__ |  |