| OP. NUMBER | Description | Times | Actual Data |
| --- | --- | --- | --- |
| 1 | Load required amount of IP.4 into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of IP.4 using balance 007-XX into a jug "TP.5 ATI". 3. Pour IP.4 into the Reactor via handhole using a plastic funnel "TP.5 ATI". 4. Addition is done - In one portion. 5. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 2 | Load required amount of Pd/C into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of Pd/C using balance 007-XX into a jug "TP.5 ATI". 3. Load Pd/C into the Reactor via handhole using a plastic scoop "TP.5 ATI". 4. Addition is done - In one portion. 5. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 3 | Start a hold time: 1. Adjust stirring rate to 300-400rpm. 2. Set/Adjust inert gas flow rate to up to 2bar, then break to atm.. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 4 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 110oC. 2. Keep the temperature of reaction mixture in range: 80-81 °C. 3. Adjust stirring rate to 300-400rpm. 4. Set/Adjust inert gas flow rate to 10-11 bar. 5. Hold for duration: 44 h. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 5 | In-process control: 1. Apply sample quenching: 10 µL RM + 950 µL MeOH. 2. Expected Result: conv. NLT99%. 3. On IPC failure: Add fresh catalyst, and continue for 12-24h. 4. On IPC pass: continue. 5. Estimated time for analysis: 1h. | Start: __:__ End: __:__ | Result: ..............; |
| 6 | 1. Remove the secondary package and carefully open the primary package. 2. Addition is done - In one portion. 3. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ |  |
| 7 | Start a heating/cooling operation: 1. Check that cooling water for heating/cooling circulator 011-XX is opened. 2. Set/Adjust temperature on thermostat 011-XX to 5-10oC. 3. Keep the temperature of reaction mixture in range: 25-35oC. 4. Provide temperature gradient: cooling straight to the required temp.. 5. Set/Adjust stirring rate to 200-400rpm. 6. Adjust inert gas flow rate to 0.5-1L/min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 8 | Filter 046-XX setup: 1. Place a PTFE seal on filtrate receiver top edge. 2. Connect suspension receiver to filtrate receiver 3. Install filter cloth into filter and fix it with the plastic ring. 4. Connect hose "TP.5 ATI" to filtrate receiver vacuum valve. Connect other end of hose to a membrane pump 001-XX. Membrane pump 001-XX connected to receiver vacuum valve:  Filtration cloth is cut and installed properly:  | Start: __:__ End: __:__ |  |
| 9 | Filter the material: 1. Set the vacuum pump to 200-300Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 10 | repeat subbatches. Keep first sub batch here in reactor @2-8oC and wait for 2nd portion | Start: __:__ End: __:__ |  |
| 11 | Load required amount of NaOH into the dropping funnel of Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of NaOH using balance 007-XX into a jug "TP.5 ATI". 3. Load NaOH into the dropping funnel using a plastic funnel "TP.5 ATI". 4. Set/Adjust stirring rate in reactor 002-XX to 105-200rpm. 5. Adjust pH to: 8-9 [WARNING, PARAMETER IS CY]. 6. Set/Adjust temperature on thermostat 011-XX to -10oC. 7. Keep the temperature of reaction mixture in range: 0…+50oC. 8. Addition is done - temperature and pH controlled. 9. Keep/Provide temperature of loaded material in range: 15-25oC. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 12 | Start a heating/cooling operation: 1. Check that cooling water for heating/cooling circulator 011-XX is opened. 2. Set/Adjust temperature on thermostat 011-XX to -15… +5oC. 3. Keep the temperature of reaction mixture in range: 5...10oC. 4. Provide temperature gradient: straight to the required temp. range. 5. Set/Adjust stirring rate to 150-200rpm. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 13 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 0…5oC. 2. Keep the temperature of reaction mixture in range: 5…10oC. 3. Adjust stirring rate to 150-200rpm. 4. Hold for duration: NLT 2h. 5. Optional: Possible pause - keep overnight at 5 … 10oC. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 14 | Filter 046-XX setup: 1. Place a PTFE seal on filtrate receiver top edge. 2. Connect suspension receiver to filtrate receiver 3. Install filter cloth into filter and fix it with the plastic ring. 4. Connect hose "TP.5 ATI" to filtrate receiver vacuum valve. Connect other end of hose to a membrane pump 001-XX. Membrane pump 001-XX connected to receiver vacuum valve:  Filtration cloth is cut and installed properly:  | Start: __:__ End: __:__ |  |
| 15 | Filter the material: 1. Set the vacuum pump to 30Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: Until Filter cake is visually dry. 4. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 16 | Wash filter cake: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use required amount of Water for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: Manually. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 17 | Filter the material: 1. Set the vacuum pump to 30Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: Until Filter cake is visually dry. 4. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 18 | Wash filter cake: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use required amount of Water for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: Manually. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 19 | Filter the material: 1. Set the vacuum pump to 30Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: Until Filter cake is visually dry. 4. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 20 | Wash filter cake: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use required amount of Water for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: Manually. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 21 | Filter the material: 1. Set the vacuum pump to 30Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: Until Filter cake is visually dry. 4. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 22 | Wash filter cake: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use required amount of iPrOH for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: Manually. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 23 | Filter the material: 1. Set the vacuum pump to 30Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: Until Filter cake is visually dry. 4. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 24 | Dry the filter cake on filter under vacuum: 1. Once Filter cake is visually dry, maintain it on the filter under vacuum. 2. Continue to apply vacuum 30Torr for 20-30min to remove residual solvents. 3. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. 4. Consider air, moisture, and light sensitivity of the material. 5. After finishing drying on filter, pump the filtrate from the filter into waste canister and weigh it. m tara = ....... kg ; m gross weight = ....... kg ; m net "type of waste" waste = ....... kg ; | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 25 | Load required amount of IP.5 base into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of IP.5 base using balance 007-XX into a jug "TP.5 ATI". 3. Load IP.5 base into the Reactor via handhole using a plastic scoop "TP.5 ATI". 4. Set/Adjust temperature on thermostat 011-XX to 15-25oC. 5. Keep the temperature of reaction mixture in range: 15-25oC (not controlled). 6. Addition is done - In one portion. 7. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual temperature set : ........ oC; |
| 26 | Load required amount of Isopropanol into Reactor 002-XX: 1. Weigh the required amount of Isopropanol using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.5 ATI". 3. Using the peristaltic pump, load Isopropanol via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust temperature on thermostat 011-XX to 15-25oC. 6. Keep the temperature of reaction mixture in range: 15-25oC (not controlled). 7. Addition is done - In one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 27 | Start a heating/cooling operation: 1. Check that cooling water for heating/cooling circulator 011-XX is opened. 2. Set/Adjust temperature on thermostat 011-XX to 100->89oC. 3. Keep the temperature of reaction mixture in range: 82-83oC. 4. Provide temperature gradient: straight to the required temp.. 5. Set/Adjust stirring rate to 150-200rpm. 6. Adjust inert gas flow rate to NO. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 28 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 85-90oC. 2. Keep the temperature of reaction mixture in range: 82-83oC. 3. Adjust stirring rate to 150-200rpm. 4. Hold for duration: 1 h. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 29 | Candle filter preparation: 1. Assemble the filter using the 0.1um cartridge and filter case. 2. Secure the case with the special key. 3. Connect Norprene hoses: attach the inlet to the larger pore side and the outlet to the smaller pore side. | Start: __:__ End: __:__ |  |
| 30 | Filter the reaction mass using candle filter: 1. Set flow rate to 40-60%. | Start: __:__ End: __:__ |  |
| 31 | Prepare the reactor: | Start: __:__ End: __:__ |  |
| 32 | Load required amount of HCl in IPrOH into the dropping funnel of Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of HCl in IPrOH using balance 007-XX into a jug "TP.5 ATI". 3. Load HCl in IPrOH into the dropping funnel using a plastic funnel "TP.5 ATI". 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust temperature on thermostat 011-XX to 0…5oC. 6. Keep the temperature of reaction mixture in range: 15-60oC. 7. Addition is done - temp controlled. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 33 | Start a heating/cooling operation: 1. Check that cooling water for heating/cooling circulator 011-XX is opened. 2. Set/Adjust temperature on thermostat 011-XX to -10…0oC. 3. Keep the temperature of reaction mixture in range: 0..5oC. 4. Provide temperature gradient: straight to the required temp.. 5. Set/Adjust stirring rate to 150-200rpm. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 34 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 0oC. 2. Keep the temperature of reaction mixture in range: 0…5oC. 3. Adjust stirring rate to 150-200rpm. 4. Hold for duration: NLT 8h. 5. Optional: Possible pause - Keep overnight. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 35 | Filter 046-XX setup: 1. Place a PTFE seal on filtrate receiver top edge. 2. Connect suspension receiver to filtrate receiver 3. Install filter cloth into filter and fix it with the plastic ring. 4. Connect hose "TP.5 ATI" to filtrate receiver vacuum valve. Connect other end of hose to a membrane pump 001-XX. Membrane pump 001-XX connected to receiver vacuum valve:  Filtration cloth is cut and installed properly:  | Start: __:__ End: __:__ |  |
| 36 | Filter the material: 1. Set the vacuum pump to 30Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: Filtration complete. 4. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 37 | Wash filter cake: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use required amount of Acetone for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: thorough mixing on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 38 | Filter the material: 1. Set the vacuum pump to 30Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: Filtration complete. 4. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 39 | Wash filter cake: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use required amount of Acetone for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: thorough mixing on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 40 | Filter the material: 1. Set the vacuum pump to 30Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: Filtration complete. 4. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 41 | Wash filter cake: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use required amount of Acetone for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: thorough mixing on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 42 | Filter the material: 1. Set the vacuum pump to 30Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: Filtration complete. 4. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 43 | Dry the filter cake on filter under vacuum: 1. Once Filter cake is visually dry, maintain it on the filter under vacuum. 2. Continue to apply vacuum 30Torr for 10-15min to remove residual solvents. 3. Use a spoon "TP.5 ATI" to press and remove cracks that might appear on filter cake. 4. Consider air, moisture, and light sensitivity of the material. 5. After finishing drying on filter, pump the filtrate from the filter into waste canister and weigh it. m tara = ....... kg ; m gross weight = ....... kg ; m net "type of waste" waste = ....... kg ; | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 44 | Dry the product in oven 012-XX: 1. Check and record the tare mass of each tray into table XX.1. 2. Transfer the product (filter cake) onto trays with a layer thickness of NMT4cm. 3. Place the trays in the oven. 4. Set temperature of the oven to 40 °C with overheating protection: 60oC. 5. Adjust fan to: fan 50%. 6. Set flap to flap 50%. 7. Dry the product, checking the weight periodically (NLT 12h after start of drying) to determine dryness 20-24h. 8. Perform Mix/delump: Yes, after NLT 12h. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; |
| 45 | Finish drying: 1. Save heating/cooling circulator 011-XX temperature logs on the USB stick that is inside the control panel or in the USB ports. 2. Turn off membrane pump and repressurize the oven. Turn off the oven. 3. Remove the trays from the oven and weigh them. Calculate loss on drying and fill Table X. 4. Calculate process yield: | Start: __:__ End: __:__ |  |
| 46 | Pack the product: 1. Use analytical sample amount: 20g. 2. Retain sample amount: 40g. 3. Pack using: double PE bag + alufoil. 4. Consider air, moisture, and light sensitivity of the material: A little bit hygroscopic and light sensitive. | Start: __:__ End: __:__ |  |
| 47 | Clean the equipment: 1. Follow the solvent sequence: water, MeOH, Acetone. 2. Estimated time required: 3-5h. | Start: __:__ End: __:__ |  |
| 48 | Perform the analysis according to QDQS: 1. If analysis result is YES, proceed accordingly. 2. If analysis result is NO, repeat necessary steps. | Start: __:__ End: __:__ |  |