| OP. NUMBER | Description | Times | Actual Data |
| --- | --- | --- | --- |
| 1 | Prepare the reactor: 1. Heat the reactor to 120oC. 2. Hold the reactor at temperature for 10-20min. 3. Set Argon flow during heating to 3L/min. 4. Cool the reactor to 15-25oC. 5. Set Argon flow during cooling to 1-2L/min. | Start: __:__ End: __:__ |  |
| 2 | Load required amount of Ethanol (anhydrous) into Reactor 002-XX: 1. Weigh the required amount of Ethanol (anhydrous) using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.4 ATI". 3. Using the peristaltic pump, load Ethanol (anhydrous) via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150–200 rpm. 5. Set/Adjust inert gas flow rate to 1-2L/min. 6. Set/Adjust temperature on thermostat 011-XX to tbd. 7. Keep the temperature of reaction mixture in range: 15-25oC. 8. Addition is done - In one portion. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm Actual gas flow set: ........ L/min |
| 3 | Load required amount of IP.3 into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of IP.3 using balance 007-XX into a jug "TP.4 ATI". 3. Load IP.3 into the Reactor via handhole using a plastic scoop "TP.4 ATI". 4. Set/Adjust stirring rate in reactor 002-XX to 150–200 rpm. 5. Set/Adjust inert gas flow rate to 1-2L/min. 6. Set/Adjust temperature on thermostat 011-XX to tbd. 7. Keep the temperature of reaction mixture in range: 15-25oC. 8. Addition is done - In one portion. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm Actual gas flow set: ........ L/min |
| 4 | Start a heating/cooling operation: 1. Check that cooling water for heating/cooling circulator 011-XX is openned. 2. Set/Adjust temperature on thermostat 011-XX to -15…-10oC. 3. Keep the temperature of reaction mixture in range: -5…0°C. 4. Provide temperature gradient: straight to the target temperature. 5. Set/Adjust stirring rate to 150–200 rpm. 6. Adjust inert gas flow rate to 1-2L/min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm Actual gas flow set: ........ L/min |
| 5 | Load required amount of Sodium borohydride into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of Sodium borohydride using balance 007-XX into a jug "TP.4 ATI". 3. Load Sodium borohydride into the Reactor via handhole using a plastic scoop "TP.4 ATI". 4. Set/Adjust stirring rate in reactor 002-XX to 150–200 rpm. 5. Set/Adjust inert gas flow rate to 1-2L/min. 6. Set/Adjust temperature on thermostat 011-XX to -5...+30°C. 7. Keep the temperature of reaction mixture in range: 0…+25oC. 8. Addition is done - portionwise (temperature controlled). 9. Loading is performed during: in 1kg scale took 45min. 10. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm Actual gas flow set: ........ L/min |
| 6 | Start a heating/cooling operation: 1. Check that cooling water for heating/cooling circulator 011-XX is openned. 2. Set/Adjust temperature on thermostat 011-XX to 35 -> 15oC. 3. Keep the temperature of reaction mixture in range: 15-25oC. 4. Provide temperature gradient: straight to the target temperature. 5. Set/Adjust stirring rate to 150–200 rpm. 6. Adjust inert gas flow rate to 1-2L/min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm Actual gas flow set: ........ L/min |
| 7 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 10-20oC. 2. Keep the temperature of reaction mixture in range: 15-25oC. 3. Adjust stirring rate to 150–200 rpm. 4. Set/Adjust inert gas flow rate to 1-2L/min. 5. Hold for duration: 2-3h. 6. Optional: Possible pause - NO. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm Actual gas flow set: ........ L/min |
| 8 | In-process control: 1. Apply sample quenching: 15 µL RM + 985 µL methanol. 2. Expected Result: 99% conv.. 3. On IPC failure: 1-2h prolong the reaction.. 4. On IPC pass: continue the process. 5. Estimated time for analysis: ???. | Start: __:__ End: __:__ |  |
| 9 | Load required amount of Acetone into the dropping funnel of Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of Acetone using balance 007-XX into a jug "TP.4 ATI". 3. Load Acetone into the dropping funnel using a plastic funnel "TP.4 ATI". 4. Set/Adjust stirring rate in reactor 002-XX to 150–200 rpm. 5. Set/Adjust inert gas flow rate to 2-3L/min. 6. Set/Adjust temperature on thermostat 011-XX to -15…30oC. 7. Keep the temperature of reaction mixture in range: 0…30oC. 8. Addition is done - temp. controlled. 9. Loading is performed during: in 1kg scale took 3.5h. 10. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). 11. Optional: Possible pause - NO. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm Actual gas flow set: ........ L/min |
| 10 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 15-25oC. 2. Keep the temperature of reaction mixture in range: 15-25oC. 3. Adjust stirring rate to 70rpm. 4. Set/Adjust inert gas flow rate to 0.5-1L/min. 5. Hold for duration: 12-18h. 6. Optional: Possible pause - Yes, overnight. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm Actual gas flow set: ........ L/min |
| 11 | Load required amount of Water into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of Water using balance 007-XX into a jug "TP.4 ATI". 3. Pour Water into the Reactor via handhole using a plastic funnel "TP.4 ATI". 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust temperature on thermostat 011-XX to 65oC. 6. Keep the temperature of reaction mixture in range: 15-45oC. 7. Addition is done - In one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm |
| 12 | Start evaporation from Reactor 002-XX: 1. Set/Adjust temperature on thermostat 011-XX to 65oC with target 15-45oC. 2. Connect the membrane pump via a cold trap and turn it ON. 3. Gradually reduce the pressure within the range 400 -> 120Torr. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 4. Maintain stirring at 150-200rpm. 5. Continue until: as long as something is dripping in cold trap. Visual observation. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 13 | Load required amount of Water into Reactor 002-XX: 1. Weigh the required amount of Water using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.4 ATI". 3. Using the peristaltic pump, load Water via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust temperature on thermostat 011-XX to 20oC. 6. Keep the temperature of reaction mixture in range: 15-45oC. 7. Addition is done - In one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm |
| 14 | Load required amount of Ethyl acetate into Reactor 002-XX: 1. Weigh the required amount of Ethyl acetate using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.4 ATI". 3. Using the peristaltic pump, load Ethyl acetate via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust temperature on thermostat 011-XX to 15-25oC. 6. Keep the temperature of reaction mixture in range: 15-25oC (not controlled). 7. Addition is done - In one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm |
| 15 | Perform an extraction/separation: 1. Set/Adjust stirring in reactor to 150-200rpm. 2. Mix layers for 15-20min. 3. Stop stirring and let layers completely separate (visual check). Expected time of separation 20-30min. 4. Perform phase separation. The bottom "description" layer is stored into container "description", top "description" layer is stored in container "description"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 16 | Load required amount of Ethyl acetate into Reactor 002-XX: 1. Weigh the required amount of Ethyl acetate using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.4 ATI". 3. Using the peristaltic pump, load Ethyl acetate via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust temperature on thermostat 011-XX to 15-25oC. 6. Keep the temperature of reaction mixture in range: 15-25oC (not controlled). 7. Addition is done - In one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm |
| 17 | Perform an extraction/separation: 1. Set/Adjust stirring in reactor to 150-200rpm. 2. Mix layers for 15-20min. 3. Stop stirring and let layers completely separate (visual check). Expected time of separation 20-30min. 4. Perform phase separation. The bottom "description" layer is stored into container "description", top "description" layer is stored in container "description"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 18 | Load required amount of Sodium chloride into Reactor 002-XX: 1. Weigh the required amount of Sodium chloride using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.4 ATI". 3. Using the peristaltic pump, load Sodium chloride via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust temperature on thermostat 011-XX to 15-25oC. 6. Keep the temperature of reaction mixture in range: 15-25oC (not controlled). 7. Addition is done - In one portion. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm |
| 19 | Perform an extraction/separation: 1. Set/Adjust stirring in reactor to 150-200rpm. 2. Mix layers for 15-20min. 3. Stop stirring and let layers completely separate (visual check). Expected time of separation 20-30min. 4. Perform phase separation. The bottom "description" layer is stored into container "description", top "description" layer is stored in container "description"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 20 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to OFF. 2. Keep the temperature of reaction mixture in range: OFF. 3. Adjust stirring rate to OFF. 4. Optional: Possible pause - Yes. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 21 | Start evaporation from Rotavapor 009-XX: 1. Check Rota bath level. Add more water if needed. 2. Carefully connect and tighten Rota flask using special key. 3. Set/Adjust temperature of Rota bath 65oC. 4. Turn ON cooling water for condenser. 5. Connect the membrane pump via a cold trap and turn it ON. 6. Gradually reduce the pressure within the range 400->15Torr. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 7. Set/Adjust stirring at 50-80rpm. 8. Connect norprene hose "TP.4 ATI" for loading solution, open loading valve and load required amount of solution for evaporation. 9. Continue until: as long as something is dripping in cold trap. Visual observation. Foaming solid at the end. Be careful. Observe foaming and adjust pressure. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 22 | Load required amount of HCl in IPrOH into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of HCl in IPrOH using balance 007-XX into a jug "TP.4 ATI". 3. Pour HCl in IPrOH into the Reactor via handhole using a plastic funnel "TP.4 ATI". 4. Set/Adjust stirring rate in reactor 002-XX to 50-80rpm. 5. Set/Adjust temperature on thermostat 011-XX to 60oC. 6. Keep the temperature of reaction mixture in range: not measured (in rotavap). 7. Addition is done - in one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm |
| 23 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 60oC. 2. Keep the temperature of reaction mixture in range: not measured (in rotavap). 3. Adjust stirring rate to 50-80rpm. 4. Hold for duration: 30-50min. 5. Optional: Possible pause - Yes. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 24 | Start evaporation from Rotavapor 009-XX: 1. Check Rota bath level. Add more water if needed. 2. Carefully connect and tighten Rota flask using special key. 3. Set/Adjust temperature of Rota bath 60oC. 4. Turn ON cooling water for condenser. 5. Connect the membrane pump via a cold trap and turn it ON. 6. Gradually reduce the pressure within the range 400->15Torr. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 7. Set/Adjust stirring at 50-80rpm. 8. Connect norprene hose "TP.4 ATI" for loading solution, open loading valve and load required amount of solution for evaporation. 9. Continue until: to the dryness. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 25 | Load required amount of 2-propanol into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of 2-propanol using balance 007-XX into a jug "TP.4 ATI". 3. Pour 2-propanol into the Reactor via handhole using a plastic funnel "TP.4 ATI". 4. Set/Adjust stirring rate in reactor 002-XX to 50-80rpm. 5. Set/Adjust temperature on thermostat 011-XX to 60oC. 6. Keep the temperature of reaction mixture in range: not measured (in rotavap). 7. Addition is done - in one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm |
| 26 | Start evaporation from Rotavapor 009-XX: 1. Check Rota bath level. Add more water if needed. 2. Carefully connect and tighten Rota flask using special key. 3. Set/Adjust temperature of Rota bath 60oC. 4. Turn ON cooling water for condenser. 5. Connect the membrane pump via a cold trap and turn it ON. 6. Gradually reduce the pressure within the range 400->15Torr. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 7. Set/Adjust stirring at 50-80rpm. 8. Connect norprene hose "TP.4 ATI" for loading solution, open loading valve and load required amount of solution for evaporation. 9. Continue until: to the dryness. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 27 | Start evaporation from Rotavapor 009-XX: 1. Check Rota bath level. Add more water if needed. 2. Carefully connect and tighten Rota flask using special key. 3. Set/Adjust temperature of Rota bath 60oC. 4. Turn ON cooling water for condenser. 5. Connect the membrane pump via a cold trap and turn it ON. 6. Gradually reduce the pressure within the range NMT 30Torr. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 7. Set/Adjust stirring at 50-80rpm. 8. Connect norprene hose "TP.4 ATI" for loading solution, open loading valve and load required amount of solution for evaporation. 9. Optional: Possible pause - YES, overnight. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 28 | Load required amount of Methanesulfonic acid into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of Methanesulfonic acid using balance 007-XX into a jug "TP.4 ATI". 3. Pour Methanesulfonic acid into the Reactor via handhole using a plastic funnel "TP.4 ATI". 4. Set/Adjust stirring rate in reactor 002-XX to 50-80rpm. 5. Set/Adjust temperature on thermostat 011-XX to 60oC. 6. Keep the temperature of reaction mixture in range: not measured (in rotavap). 7. Addition is done - one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual loading: ........ kg Actual stirring set: ........ rpm |
| 29 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 60oC. 2. Keep the temperature of reaction mixture in range: not measured (in rotavap). 3. Adjust stirring rate to 50-80rpm. 4. Hold for duration: as long as needed to dissolve everything.. 5. Optional: Possible pause - YES, overnight. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm |
| 30 | Clean the equipment: | Start: __:__ End: __:__ |  |
| 31 | Perform waste treatment: | Start: __:__ End: __:__ |  |