| OP. NUMBER | Description | Times | Actual Data |
| --- | --- | --- | --- |
| 1 | Reactor 002-XX preparation: 1. Hold Reactor at temperature for 10min. 2. Set Argon flow during heating to 15-20 L/min. | Start: __:__ End: __:__ |  |
| 2 | Load Isopropyl acetate into reactor 002-XX: 1. Weigh Isopropyl acetate on balance 007-XX 2. Transfer Isopropyl acetate into reactor with peristaltic pump and hose "TP.1 ATI". Set peristaltic pump to 50-90%. 3. Set/Adjust stirring rate in reactor to 150-200rpm (recommended range). 4. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 5. Keep reaction mixture temperature in range: 15-25oC. 6. Addition speed - in one portion. 7. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: prepare reactor, purge with argon. Discuss amount of solvent to avoid or reduce evaporation | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual peristaltic pump set: ........ %; Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 3 | Load Phthalide (SM.1) into reactor 002-XX: 1. Remove secondary package and open primary package. 2. Weigh Phthalide (SM.1) on balance 007-XX into jug "TP.1 ATI". 3. Load Phthalide (SM.1) into reactor via handhole using a plastic scoop and plastic funnel "TP.1 ATI". 4. Set/Adjust stirring rate in reactor to 150-200rpm (recommended range). 5. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 6. Keep reaction mixture temperature in range: 15-25oC. 7. Addition speed - in one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: material is not dusty | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 4 | Load ImCHO-DMSA into reactor 002-XX: 1. Remove secondary package and open primary package. 2. Weigh ImCHO-DMSA on balance 007-XX into jug "TP.1 ATI". 3. Load ImCHO-DMSA into reactor via handhole using a plastic scoop and plastic funnel "TP.1 ATI". 4. Set/Adjust stirring rate in reactor to 150-200rpm (recommended range). 5. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 6. Keep reaction mixture temperature in range: 15-25oC. 7. Addition speed - in one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: material is not dusty | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 5 | Load Isopropyl acetate into reactor 002-XX: 1. Remove secondary package and open primary package. 2. Weigh Isopropyl acetate on balance 007-XX into jug "TP.1 ATI". 3. Pour Isopropyl acetate into reactor via handhole using a plastic funnel "TP.1 ATI". 4. Set/Adjust stirring rate in reactor to 150-200rpm (recommended range). 5. Set/Adjust temperature on heating/cooling circulator 011-XX to 20oC (keep jacket/RM temp. difference below 50oC). 6. Keep reaction mixture temperature in range: 15-25oC. 7. Addition speed - in one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 6 | Load NaOMe (30% in MeOH) into reactor 002-XX: 1. Weigh NaOMe (30% in MeOH) on balance 007-XX 2. Transfer NaOMe (30% in MeOH) into reactor with peristaltic pump and hose "TP.1 ATI". Set peristaltic pump to 10-20%. 3. Set/Adjust stirring rate in reactor to 150-200 (recommended range). 4. Set/Adjust temperature on heating/cooling circulator 011-XX to 0oC (keep jacket/RM temp. difference below 50oC). 5. Keep reaction mixture temperature in range: 15-35oC. 6. Addition speed - addition is temperatue controlled. 7. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: 1. mild exotherm 2. in tech batch took 1h this reagent reacts with moisture | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual peristaltic pump set: ........ %; Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 7 | Load Isopropyl acetate into reactor 002-XX: 1. Remove secondary package and open primary package. 2. Weigh Isopropyl acetate on balance 007-XX into jug "TP.1 ATI". 3. Pour Isopropyl acetate into reactor via handhole using a plastic funnel "TP.1 ATI". 4. Set/Adjust stirring rate in reactor to 150-200 (recommended range). 5. Set/Adjust temperature on heating/cooling circulator 011-XX to 10-15oC (keep jacket/RM temp. difference below 50oC). 6. Keep reaction mixture temperature in range: 15-35oC. 7. Addition speed - in one portion. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: use to rinse the hose and reactor's walls from inside | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 8 | Start heating/cooling with heating/cooling circulator 011-XX: 1. Check that cooling water for 011-XX is opened. 2. Set/Adjust temperature on 011-XX to tbd->27oC (keep jacket/RM temp. difference below 50oC). 3. Keep reaction mixture temperature in range: 22-35oC. 4. Set/Adjust stirring in reactor 002-XX to 150-200 (recommended range). 5. Set/Adjust inert gas flow rate to 1-2 L/min. Fill Table X at least every Y minutes. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 9 | Start a hold time: 1. Set/Adjust heating/cooling circulator 011-XX to 27oC. 2. Keep reaction mixture temperature in range: 22-35oC for NMT 2h. 3. Set/Adjust stirring in reactor 002-XX to 150-200rpm (recommended range). 4. Set/Adjust inert gas flow rate to 1-2 L/min. Fill Table X at least every Y minutes. COMMENT INFO: NMT2h from reaction started; start decomp. Reaction was done at reflux as well and it works. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 10 | In-process control: In-process control (determination of IP.X conversion after 1-1.5h): 1. Take approximately 1mL of reaction mixture into a 20 mL vial; 2. Purge the vial with argon; 3. Label the sample with an IPC number. For example: TBD-0XXX-Y IP.X RM IPC 1, where Y is the batch number (copied from the header) and X is the IPC serial number. 4. Submit the sample to QC for analysis. 5. Keep stirring the reaction mixture until the IPC result is available. 6. Estimated time for analysis: 0.5h Decision criteria based on IPC result: • If IP.X conversion is conv. 95%, proceed with the workup (Operation XX). • On IPC failure: continue process with current conv. • On IPC pass: continue. IPC 1: TBD-0XXX-Y IP.X RM IPC 1 Results: Corresponds to RelS requirements:  - Yes  - No IP.X conversion: ..... % COMMENT INFO: after 1-1.5h. 0.5h hplc run what do we do then? - No need to add anything else; presence of sms is not critical. They are purgible. Next step reaction is still cont. in the future can be removed (no need for IPC) take IPC from agitated RM | Start: __:__ End: __:__ | Result: ..............; |
| 11 | Start evaporation in reactor 002-XX: 1. Set/Adjust temperature heating/cooling circulator 011-XX to 80->65oC with target 40-50oC (keep jacket/RM temp. difference below 50oC). 2. Connect a membrane pump via a cold trap and turn it ON. 3. Gradually reduce pressure NLT 100Torr. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 4. Set/Adjust stirring to 150-200rpm (recommended range). 5. Continue until: 50-60% is out. Fill Table X at least every Y minutes. COMMENT INFO: evap from the reactor is ok. We can remove 50% of solvent in 3kg scale in 2.5h. 80% or more. More acetate is left - worse precipitate is formed. This 80% for initial rm volume it was tested distillation at pH 14; 7 and 1-2; In case of 14 and 7 the material is stable. during tech batch, we removed slightly above 50% ofthe volume | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual pressure: ........ Torr; Actual stirring set: ........ rpm; |
| 12 | Load Water into reactor 002-XX: 1. Weigh Water on balance 007-XX 2. Transfer Water into reactor with peristaltic pump and hose "TP.1 ATI". Set peristaltic pump to 10-90%. 3. Set/Adjust stirring rate in reactor to 150-200 (recommended range). 4. Set/Adjust temperature on heating/cooling circulator 011-XX to 0oC (keep jacket/RM temp. difference below 50oC). 5. Keep reaction mixture temperature in range: 15-30oC. 6. Addition speed - First 3-6kg are added portionwise. Temperature controlled. 7. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: rm is susp. Rnd can check if we need so much of water | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual peristaltic pump set: ........ %; Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 13 | Load Hydrochloric acid 35% into Reactor 002-XX dropping funnel: 1. Remove secondary package and open primary package. 2. Weigh Hydrochloric acid 35% on balance 007-XX into jug "TP.1 ATI". 3. Load Hydrochloric acid 35% into the dropping funnel using a plastic funnel "TP.1 ATI". 4. Set/Adjust stirring rate in reactor to 150-200 (recommended range). 5. Adjust pH to: 1-2, should be recheced after hold time. 6. Set/Adjust temperature on heating/cooling circulator 011-XX to start with-10 at the end adjust 15oC (keep jacket/RM temp. difference below 50oC). 7. Keep reaction mixture temperature in range: in range 0 … +40oC. 8. Addition speed - portionwise, temperature and pH controlled. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Fill Table X at least every Y minutes. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: product should precipitate, expected time of adj in 3kg scale was 35min. in tech batch ca 1h check amount of HCl | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 14 | Start a hold time: 1. Set/Adjust heating/cooling circulator 011-XX to 15oC. 2. Keep reaction mixture temperature in range: 10-25oC for NLT 40min. 3. Set/Adjust stirring in reactor 002-XX to 100-150rpm (recommended range). 4. Optional: Possible pause - Yes, overnight. Fill Table X at least every Y minutes. COMMENT INFO: pH must be checked one more time after the hold time. If it is overnight, check pH in the morning. for overnight Set temp to 10oC. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 15 | Filter 046-6-7 setup: 1. Place a PTFE seal on filtrate receiver top edge. 2. Connect suspension receiver to filtrate receiver 3. Install filter cloth into filter and fix it with the plastic ring. 4. Connect hose "TP.1 ATI" to filtrate receiver vacuum valve. Connect other end of hose to a membrane pump 001-XX. Membrane pump 001-XX connected to receiver vacuum valve:  Filtration cloth is cut and installed properly:  Filter 046-2-13 setup: 5. Connect suspension receiver to glass receiver from reactor 002-XX with norprene hose "TP.1 ATI". 6. Connect reactor receiver to membrane pump 001-XX with norprene hose "vacuum". 7. Install filter cloth into filter and fix it with the plastic ring. Filter is connected to Reactor's receiver vessel:  Membrane pump 001-XX is connected to Reactor's receiver vacuum valve:  Filtration cloth is cut and installed properly:  | Start: __:__ End: __:__ |  |
| 16 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.1 ATI" from reactor valve and through peristaltic pump 001-XX to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Set the vacuum pump to 300-350Torr and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Continue until: Until product is dry on filter. 6. Use a spoon "TP.1 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "TP.1 ATI". Resume the process afterwards. COMMENT INFO: using reactor as receiver. Product can be pulled from the bottom valve. (used 046-2 with reactor as filtr. Bottle). HERE, we need large filter!!! | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 17 | Dry the filter cake on filter under vacuum: 1. Once Filter cake is visually dry, maintain it on the filter under vacuum. 2. Continue to apply vacuum 300-350Torr for 30-50min to remove residual solvents. 3. Use a spoon "TP.1 ATI" to press and remove cracks that might appear on filter cake. 4. Consider air, moisture, and light sensitivity of the material. 5. After finishing drying on filter, pump the filtrate from the filter into waste canister and weigh it. m tara = ....... kg ; m gross weight = ....... kg ; m net "type of waste" waste = ....... kg ; | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 18 | Wash filter cake with 2-propanol: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use 2-propanol for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: manual mixing of product on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 19 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.1 ATI" from reactor valve and through peristaltic pump 001-XX to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Set the vacuum pump to 300-350Torr and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Continue until: Until product is dry on filter. 6. Use a spoon "TP.1 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "TP.1 ATI". Resume the process afterwards. COMMENT INFO: using reactor as receiver. Product can be pulled from the bottom valve. (used 046-2 with reactor as filtr. Bottle). HERE, we need large filter!!! | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 20 | Wash filter cake with Water: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use Water for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: manual mixing of product on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: pH of the product itself is 1-2, so checking pH of filtrate is pointless. | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 21 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.1 ATI" from reactor valve and through peristaltic pump 001-XX to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Set the vacuum pump to 300-350Torr and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Continue until: Until product is dry on filter. 6. Use a spoon "TP.1 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "TP.1 ATI". Resume the process afterwards. COMMENT INFO: using reactor as receiver. Product can be pulled from the bottom valve. (used 046-2 with reactor as filtr. Bottle). HERE, we need large filter!!! | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 22 | Wash filter cake with Water: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use Water for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: manual mixing of product on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 23 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.1 ATI" from reactor valve and through peristaltic pump 001-XX to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Set the vacuum pump to 300-350Torr and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Continue until: Until product is dry on filter. 6. Use a spoon "TP.1 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "TP.1 ATI". Resume the process afterwards. COMMENT INFO: using reactor as receiver. Product can be pulled from the bottom valve. (used 046-13 with reactor as filtr. Bottle). HERE, we need large filter!!! | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 24 | Wash filter cake with Water: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use Water for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: manual mixing of product on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 25 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.1 ATI" from reactor valve and through peristaltic pump 001-XX to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Set the vacuum pump to 300-350Torr and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Continue until: Until product is dry on filter. 6. Use a spoon "TP.1 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "TP.1 ATI". Resume the process afterwards. COMMENT INFO: using reactor as receiver. Product can be pulled from the bottom valve. (used 046-13 with reactor as filtr. Bottle). HERE, we need large filter!!! | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 26 | Wash filter cake with 2-propanol: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use 2-propanol for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: manual mixing of product on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: during pre-val check first and second wash. | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 27 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.1 ATI" from reactor valve and through peristaltic pump 001-XX to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Set the vacuum pump to 300-350Torr and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Continue until: Until product is dry on filter. 6. Use a spoon "TP.1 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "TP.1 ATI". Resume the process afterwards. COMMENT INFO: using reactor as receiver. Product can be pulled from the bottom valve. (used 046-2 with reactor as filtr. Bottle). HERE, we need large filter!!! | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 28 | Wash filter cake with 2-propanol: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use 2-propanol for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: manual mixing of product on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: during pre-val check first and second wash. | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 29 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.1 ATI" from reactor valve and through peristaltic pump 001-XX to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Set the vacuum pump to 300-350Torr and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Continue until: Until product is dry on filter. 6. Use a spoon "TP.1 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "TP.1 ATI". Resume the process afterwards. COMMENT INFO: using reactor as receiver. Product can be pulled from the bottom valve. (used 046-2 with reactor as filtr. Bottle). HERE, we need large filter!!! | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 30 | Dry the filter cake on filter under vacuum: 1. Once Filter cake is visually dry, maintain it on the filter under vacuum. 2. Continue to apply vacuum 300-500Torr for 30-50min to remove residual solvents. 3. Use a spoon "TP.1 ATI" to press and remove cracks that might appear on filter cake. 4. Consider air, moisture, and light sensitivity of the material. 5. After finishing drying on filter, pump the filtrate from the filter into waste canister and weigh it. m tara = ....... kg ; m gross weight = ....... kg ; m net "type of waste" waste = ....... kg ; | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 31 | Dry the product in oven 012-XX: 1. Check and record the tare mass of each tray into table XX.1. 2. Transfer the product (filter cake) onto trays with a layer thickness of NMT 4cm. Divide all wet product approximately evenly between all trays. Crush big lumps with shovel. 3. Place the trays in the oven. 4. Set temperature of the oven to 50oC with overheating protection: 60oC. 5. Adjust fan to: "70%". 6. Set flap to "70%". 7. Dry the product, checking the weight periodically (NLT 12h after start of drying) to determine dryness 24-48h. 8. Perform Mix/delump: yes, after NLT 12h. Record weights in Table X. Table X filled:  Overheating set: .......°C Actual fan set: .......%; Actual flap set: .......%; Temperature after at least 1h from the start: .......°C COMMENT INFO: in rnd conduct test of drying at 60oC in 3kg tech batch prod after iPrOH was dry after 24h. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; |
| 32 | Finish drying: 1. Save heating/cooling circulator 011-XX temperature logs on the USB stick that is inside the control panel or in the USB ports. 2. Turn off membrane pump and repressurize the oven. Turn off the oven. 3. Remove the trays from the oven and weigh them. Calculate loss on drying and fill Table X. 4. Calculate process yield: | Start: __:__ End: __:__ |  |
| 33 | after drying prod. Is sampled and packed. | Start: __:__ End: __:__ |  |
| 34 | Analyzing, correspond to QDQS | Start: __:__ End: __:__ |  |
| 35 | Reactors and filters are cleaned | Start: __:__ End: __:__ |  |
| 36 | Perform waste treatment: | Start: __:__ End: __:__ |  |