| OP. NUMBER | Description | Times | Actual Data |
| --- | --- | --- | --- |
| 1 | Prepare Reactor 002-XX: | Start: __:__ End: __:__ |  |
| 2 | Load Methanol into reactor 002-XX: 1. Weigh Methanol using balance 007-XX. 2. Connect peristaltic pump and hose "TP.3 ATI". 3. Using peristaltic pump, load Methanol via handhole. Secure hose to handhole with plastic tie. 4. Set peristaltic pump to 50-90%. 5. Stirring rate in reactor 130-170rpm (range is recommended and can be adjusted). 6. Set/Adjust temperature on thermostat to 20°C (keep jacket/RM temp. difference below 50oC). 7. Keep reaction mixture temperature in range: 15-25°C. 8. Addition speed - in one portioin. 9. Keep/Provide temperature of loaded material in range: 15-25°C (not controlled). Record parameters into Table X every YY min or more often. Specified loading: ….. kg (… - … kg) COMMENT INFO: used this amount to susp. IP.2 for loading | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) ......... kg Peristaltic pump: ........ %; Stirring: ........ rpm; Temperature set : ........ °C; |
| 3 | Load IP.2 into reactor 002-XX: 1. Weigh IP.2 on balance 007-XX into jug "TP.3 ATI" in ca Xkg portion. 2. Load IP.2 into reactor via handhole using a plastic scoop and plastic funnel "TP.3 ATI". 3. Stirring rate in reactor 150-200rpm (range is recommended and can be adjusted). 4. Set/Adjust temperature on thermostat to 20°C (keep jacket/RM temp. difference below 50oC). 5. Keep reaction mixture temperature in range: 15-25°C. 6. Addition speed - in one portioin. 7. Keep/Provide temperature of loaded material in range: 15-25°C (not controlled). Record parameters into Table X every YY min or more often. Specified loading: ….. kg (… - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) ......... kg Stirring: ........ rpm; Temperature set : ........ °C; |
| 4 | Load Hydrochloric acid 35% into reactor 002-XX: 1. Weigh Hydrochloric acid 35% using balance 007-XX. 2. Connect peristaltic pump and hose "TP.3 ATI". 3. Using peristaltic pump, load Hydrochloric acid 35% via handhole. Secure hose to handhole with plastic tie. 4. Set peristaltic pump to 60-80%. 5. Stirring rate in reactor 150-200rpm (range is recommended and can be adjusted). 6. Set/Adjust temperature on thermostat to 20°C (keep jacket/RM temp. difference below 50oC). 7. Keep reaction mixture temperature in range: 15-25°C. 8. Addition speed - in one portioin. 9. Keep/Provide temperature of loaded material in range: 15-25°C (not controlled). Record parameters into Table X every YY min or more often. Specified loading: ….. kg (… - … kg) COMMENT INFO: loaded with vacuum | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) ......... kg Peristaltic pump: ........ %; Stirring: ........ rpm; Temperature set : ........ °C; |
| 5 | Heating/Cooling with thermostat 011-XX: 1. Check that cooling water for 011-XX is opened. 2. Set/Adjust temperature on 011-XX to 75-80°C (keep jacket/RM temp. difference below 50oC). 3. Keep reaction mixture temperature in range: 65-69°C (mild reflux). 4. Provide temperature gradient: straight. 5. Stirring in reactor 002-XX 150-200rpm (range is recommended and can be adjusted). Record parameters into Table X every YY min or more often. COMMENT INFO: use reciever of reactor filled with water (make a task in CU) try subboiling - we tried - too long reaction time | Start: __:__ End: __:__ | Temperature set : ........ °C; Temperature of reaction mixture: ........ °C; Stirring: ........ rpm; |
| 6 | Hold time: 1. Ensure that temperature on thermostat is set to 75-80°C. 2. Hold temperature of reaction mixture in range: 67-70°C for NLT 3h. 3. Stirring in reactor 002-XX 150-200rpm (range is recommended and can be adjusted). 4. Optional: Keep reaction mixture overnight - Yes, overnight at room temp. Record parameters into Table X every YY min or more often. COMMENT INFO: sub-boiling extends reaction time too much longer time is not a problem cool reaction mixture and can have a break point | Start: __:__ End: __:__ | Temperature set : ........ °C; Temperature of reaction mixture: ........ °C; Stirring: ........ rpm; |
| 7 | Prepare "description" solution according to table 1.X: Solution is prepared:  | Start: __:__ End: __:__ |  |
| 8 | In-process control: In-process control (determination of IP.X conversion after NLT 3h): 1. Take approximately 1mL of reaction mixture into a 20 mL vial; 2. Purge the vial with argon; 3. Label the sample with an IPC number. 4. Submit the sample to QC for analysis. 5. Keep stirring the reaction mixture until the IPC result is available. 6. Estimated time for analysis: ~1h Decision criteria based on IPC result (conversion is critical to yield): • If conversion is conv. NLT99% WARNING, PARAMETER IS PC, proceed with the workup (Operation XX). coninue • On IPC failure: continue heating for around 1-2h more and repeat IPC. IPC 1: TBD-0XXX-.... IP.X RM IPC 1 Conversion: ..... % COMMENT INFO: will need to cool the rm before taking sample (rm to 50oC). Start to cool after 3h. After sample is taken, heat it up back and continue to wait ipc result. in 3kg scale we got 100% conv in 3h unprotected sm will cause problems with purity rm can stay overnight at room temp. | Start: __:__ End: __:__ | Result: ..............; |
| 9 | Heating/Cooling with thermostat 011-XX: 1. Check that cooling water for 011-XX is opened. 2. Set/Adjust temperature on 011-XX to 0->15°C (keep jacket/RM temp. difference below 50oC). 3. Keep reaction mixture temperature in range: 15-25°C. 4. Provide temperature gradient: straight cooling to the required temperature.. 5. Stirring in reactor 002-XX 150-200rpm (range is recommended and can be adjusted). Record parameters into Table X every YY min or more often. COMMENT INFO: after cooling we can have a break point | Start: __:__ End: __:__ | Temperature set : ........ °C; Temperature of reaction mixture: ........ °C; Stirring: ........ rpm; |
| 10 | Load Sodium hydroxide into Reactor 002-XX dropping funnel: 1. Weigh Sodium hydroxide on balance 007-XX into jug "TP.3 ATI". 2. Load Sodium hydroxide into the dropping funnel using a plastic funnel "TP.3 ATI". 3. Stirring rate in reactor 150-200rpm (range is recommended and can be adjusted). 4. Adjust pH to: 8-9 WARNING, PARAMETER IS PC. Measurement is done after 3-5 min from last addition. 5. Set/Adjust temperature on thermostat to -10…-15°C (keep jacket/RM temp. difference below 50oC). 6. Keep reaction mixture temperature in range: 0-30°C. 7. Addition speed - addition is temperature controlled. Record parameters into Table X every YY min or more often. Specified loading: ….. kg (… - … kg) COMMENT INFO: in 3kg scale took 1h, start pH measurment after 50-80% of NaOH is added (1eq to HCl) HCl can be added to correct the pH make 2 operations for thermo control and pH control at first rm is dark, after neutr it become off white, light colour <8 bad yield; >9 purity can be affected; range is potentially critical parameter use paper in addition for validation only | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) ......... kg Stirring: ........ rpm; Temperature set : ........ °C; |
| 11 | Heating/Cooling with thermostat 011-XX: 1. Check that cooling water for 011-XX is opened. 2. Set/Adjust temperature on 011-XX to -5…0°C (keep jacket/RM temp. difference below 50oC). 3. Keep reaction mixture temperature in range: 10-15°C. 4. Provide temperature gradient: straight cooling to the required temperature.. 5. Stirring in reactor 002-XX 150-200rpm (range is recommended and can be adjusted). Record parameters into Table X every YY min or more often. COMMENT INFO: keeping in mind 50°C delta between the jacket temp. and rm temp after cooling we can have a break point | Start: __:__ End: __:__ | Temperature set : ........ °C; Temperature of reaction mixture: ........ °C; Stirring: ........ rpm; |
| 12 | Hold time: 1. Ensure that temperature on thermostat is set to 5…15°C. 2. Hold temperature of reaction mixture in range: 10-15°C for 40-50min. 3. Stirring in reactor 002-XX 150-200 (range is recommended and can be adjusted). 4. Optional: Keep reaction mixture overnight - YES, when pH is 8-9. Record parameters into Table X every YY min or more often. COMMENT INFO: after hold time check pH and adjust it again to 8-9 | Start: __:__ End: __:__ | Temperature set : ........ °C; Temperature of reaction mixture: ........ °C; Stirring: ........ rpm; |
| 13 | Filter 046-6-7 (SS filter) setup: 1. Place a PTFE seal on filtrate receiver top edge. 2. Connect suspension receiver to filtrate receiver 3. Install filter cloth into filter and fix it with the plastic ring. 4. Connect hose "TP.3 ATI" to filtrate receiver vacuum valve. Connect other end of hose to a membrane pump. Membrane pump connected to receiver vacuum valve:  Filtration cloth is cut and installed properly:  Filter 046-2-13 (PP filter) setup: 5. Connect suspension receiver to glass receiver from reactor 002-XX with norprene hose "TP.3 ATI". 6. Connect reactor receiver to membrane pump with norprene hose "vacuum". 7. Install filter cloth into filter and fix it with the plastic ring. Filter is connected to Reactor's receiver vessel:  Membrane pump is connected to Reactor's receiver vacuum valve:  Filtration cloth is cut and installed properly:  | Start: __:__ End: __:__ |  |
| 14 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.3 ATI" from reactor valve and through peristaltic pump to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 4. Continue until when Filter Cake is dry. 5. Use a spoon "TP.3 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into container "TP.3 ATI". Resume the process afterwards. COMMENT INFO: on filter 046-2 or 046-13 with reactor as filtr bottle | Start: __:__ End: __:__ |  |
| 15 | Wash filter cake with Water: 1. Stop membrane pump. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Weight Water for washing Filter cake using balance 007-XX. Then pour onto filter cake. 3. Ensure loaded material temperature is 15-25°C (not controlled). 4. Mix on filter manually: manual on filter. Specified loading: ….. kg (… - … kg) COMMENT INFO: manual mixint on filter | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) ......... kg |
| 16 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.3 ATI" from reactor valve and through peristaltic pump to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 4. Continue until when Filter Cake is dry. 5. Use a spoon "TP.3 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into container "TP.3 ATI". Resume the process afterwards. | Start: __:__ End: __:__ |  |
| 17 | Wash filter cake with Water: 1. Stop membrane pump. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Weight Water for washing Filter cake using balance 007-XX. Then pour onto filter cake. 3. Ensure loaded material temperature is 15-25°C (not controlled). Specified loading: ….. kg (… - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) ......... kg |
| 18 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.3 ATI" from reactor valve and through peristaltic pump to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 4. Continue until when Filter Cake is dry. 5. Use a spoon "TP.3 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into container "TP.3 ATI". Resume the process afterwards. | Start: __:__ End: __:__ |  |
| 19 | Dry filter cake on filter under vacuum: 1. Once Filter cake is visually dry, maintain it on the filter. 2. Use a spoon "TP.3 ATI" to press and remove cracks that might appear on filter cake. 3. Consider air, moisture, and light sensitivity of the material. 4. After finishing drying on filter, pump the filtrate from the filter into waste container and weigh it. m tara = ....... kg ; m gross weight = ....... kg ; m net "type of waste" waste = ....... kg ; | Start: __:__ End: __:__ |  |
| 20 | Wash filter cake with 2-propanol: 1. Stop membrane pump. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Weight 2-propanol for washing Filter cake using balance 007-XX. Then pour onto filter cake. 3. Ensure loaded material temperature is 15-25°C (not controlled). Specified loading: ….. kg (… - … kg) COMMENT INFO: check reactor with 10L | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) ......... kg |
| 21 | Hold time: 1. Ensure that temperature on thermostat is set to tbd. 2. Hold temperature of reaction mixture in range: 0-4°C WARNING, PARAMETER IS CY for 20-30min. 3. Stirring in reactor 002-XX 50-80rpm (range is recommended and can be adjusted). Record parameters into Table X every YY min or more often. COMMENT INFO: higher that 4°C - we can have yield problem. 4°C and higher is Critical to yield | Start: __:__ End: __:__ | Temperature set : ........ °C; Temperature of reaction mixture: ........ °C; Stirring: ........ rpm; |
| 22 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.3 ATI" from reactor valve and through peristaltic pump to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 4. Continue until when FK is dry.. 5. Use a spoon "TP.3 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into container "TP.3 ATI". Resume the process afterwards. | Start: __:__ End: __:__ |  |
| 23 | Load 2-propanol into reactor 002-XX: 1. Weigh 2-propanol using balance 007-XX into jug "TP.3 ATI". 2. Pour 2-propanol into reactor via handhole using a plastic funnel "TP.3 ATI". 3. Addition speed - addition in one portion. Record parameters into Table X every YY min or more often. Specified loading: ….. kg (… - … kg) COMMENT INFO: use this amount to rinse the reactor first | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) ......... kg |
| 24 | Filter the material: 1. Keep stirring in Reactor 002-XX to 150-60rpm (gradually decreasing). 2. Connect norprene hose "TP.3 ATI" from reactor valve and through peristaltic pump to Nutsch filter. Set the peristaltic pump to 40-60%. 3. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 4. Continue until when FK is dry.. 5. Use a spoon "TP.3 ATI" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into container "TP.3 ATI". Resume the process afterwards. | Start: __:__ End: __:__ |  |
| 25 | Dry filter cake on filter under vacuum: 1. Once Filter cake is visually dry, maintain it on the filter. 2. Use a spoon "TP.3 ATI" to press and remove cracks that might appear on filter cake. 3. Consider air, moisture, and light sensitivity of the material. 4. After finishing drying on filter, pump the filtrate from the filter into waste container and weigh it. m tara = ....... kg ; m gross weight = ....... kg ; m net "type of waste" waste = ....... kg ; | Start: __:__ End: __:__ |  |
| 26 | a) Weighing of empty trays for drying: 1. Weigh each tray. Record tare mass in Table X. Table X. filled:  ___________________________ b) Filling the trays 2. Transfer the wet product (Filter cake) onto trays: spread uniform layer thickness on the trays, the product should not stick out above tray edges. 3. Distribute the wet product approximately evenly among XX trays. 4. Crush any large lumps with a shovel or suitable utensil. 5. Weigh each tray with product. Record loaded mass in Table X. ___________________________ c) Preparation of Vacuum Oven 012-XX: 6. Arrange trays inside the oven, ensuring proper spacing/airflow. 7. Attach membrane pump to the oven via cold trap filled with dry ice. 8. Working temperature set point: 40-45°C. 9. Initial vacuum set point: 30Torr. NB! Do not forget to put logger on one of the middle shelves. Temperature logger is placed in oven  _____________________________ d) Drying and Interim weighings: 10. Begin drying under set conditions (40-45°C, 30Torr). 11. After NLT 12 hours of drying, perform mixing/delumping to break up any lumps. 12. Dry the product for a total of 16-24h. 13. Interim weighings: After NLT 24 h, start checking product weight periodically to determine drying progress. Fill Table X. 14. End of drying: In 2–3 hours total weight loss AND weight loss of each tray should be NMT3%. If within acceptance range – drying is over. (Proceed with the next operation.) Table X filled:  Pressure after at least 1h from the start: ...... Torr Temperature after at least 1h from the start: .......°C | Start: __:__ End: __:__ | Temperature set : ........ °C; Pressure set: ........ Torr; |
| 27 | a) Weighing of empty trays for drying: 1. Weigh each tray. Record tare mass in Table X. Table X. filled:  ___________________________ b) Filling the trays 2. Transfer the wet product (Filter cake) onto trays: spread uniform layer thickness on the trays, the product should not stick out above tray edges. 3. Distribute the wet product approximately evenly among XX trays. 4. Crush any large lumps with a shovel or suitable utensil. 5. Weigh each tray with product. Record loaded mass in Table X. ___________________________ c) Preparation of Vacuum Oven 012-XX: 6. Arrange trays inside the oven, ensuring proper spacing/airflow. 7. Attach membrane pump to the oven via cold trap filled with dry ice. 8. Working temperature set point: 40-45°C. 9. Initial vacuum set point: actual NMT2Torr. NB! Do not forget to put logger on one of the middle shelves. Temperature logger is placed in oven  _____________________________ d) Drying and Interim weighings: 10. Begin drying under set conditions (40-45°C, actual NMT2Torr). 11. After NLT 12 hours of drying, perform mixing/delumping to break up any lumps. 12. Dry the product for a total of 16-24h. 13. Interim weighings: After NLT 24 h, start checking product weight periodically to determine drying progress. Fill Table X. 14. End of drying: In 2–3 hours total weight loss AND weight loss of each tray should be NMT3%. If within acceptance range – drying is over. (Proceed with the next operation.) Table X filled:  Pressure after at least 1h from the start: ...... Torr Temperature after at least 1h from the start: .......°C COMMENT INFO: expected white - off white solid crystals product is not air/moisture sensitive performe weight check rnd will check temp stability @60°C | Start: __:__ End: __:__ | Temperature set : ........ °C; Pressure set: ........ Torr; |
| 28 | Finish drying: 1. Save thermostat 011-XX temperature logs on the USB stick that is inside the control panel or in the USB ports. 2. Turn off membrane pump and repressurize the oven. Turn off the oven. 3. Remove the trays from the oven and weigh them. Calculate loss on drying and fill Table X. 4. Calculate process yield: | Start: __:__ End: __:__ |  |
| 29 | after drying prod. Is sampled and packed. | Start: __:__ End: __:__ |  |
| 30 | the analysis according to QDQS: 1. If analysis result is YES, proceed accordingly. 2. If analysis result is NO, repeat necessary steps. | Start: __:__ End: __:__ |  |
| 31 | Reactors and filters are cleaned | Start: __:__ End: __:__ |  |
| 32 | waste treatment: | Start: __:__ End: __:__ |  |