| OP. NUMBER | Description | Times | Actual Data |
| --- | --- | --- | --- |
| 1 | Prepare the reactor: 1. Heat the reactor to 120oC. 2. Hold the reactor at temperature for 10-15min. 3. Set Argon flow during heating to 5-10 L/min. 4. Cool the reactor to 15-25oC. 5. Set Argon flow during cooling to 1-2 L/min. COMMENT INFO: visually check the condenser for condensate before drying | Start: __:__ End: __:__ |  |
| 2 | Load required amount of DCM into Reactor 002-XX: 1. Weigh the required amount of DCM using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.1 DETO". 3. Using the peristaltic pump, load DCM via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust inert gas flow rate to 1-2 L/min. 6. Set/Adjust temperature on thermostat 011-XX to 20oC. 7. Keep the temperature of reaction mixture in range: 15-25oC. 8. Addition is done - in one portion. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 3 | Load required amount of Imidazole carboxaldehyde (DETO SM.2) into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of Imidazole carboxaldehyde (DETO SM.2) using balance 007-XX into a jug "TP.1 DETO". 3. Load Imidazole carboxaldehyde (DETO SM.2) into the Reactor via handhole using a plastic scoop "TP.1 DETO". 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust inert gas flow rate to 1-2 L/min. 6. Set/Adjust temperature on thermostat 011-XX to 20oC. 7. Keep the temperature of reaction mixture in range: 15-25oC. 8. Addition is done - in portions. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: dusty solid, consider loading in 1kg portions. | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 4 | Load required amount of DMSA-Cl into the dropping funnel of Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of DMSA-Cl using balance 007-XX into a jug "TP.1 DETO". 3. Load DMSA-Cl into the dropping funnel using a plastic funnel "TP.1 DETO". 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust inert gas flow rate to 2-5 L/min. 6. Set/Adjust temperature on thermostat 011-XX to 20oC. 7. Keep the temperature of reaction mixture in range: +20… +30oC [WARNING, PARAMETER IS PC]. 8. Addition is done - pump with perist pump, temp controlled. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: liquid. Keep condenser running and control temp. consider using diaphragma pump instead of drop funnel??? | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 5 | Load required amount of DCM into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of DCM using balance 007-XX into a jug "TP.1 DETO". 3. Pour DCM into the Reactor via handhole using a plastic funnel "TP.1 DETO". 4. Set/Adjust stirring rate in reactor 002-XX to 150-200. 5. Set/Adjust inert gas flow rate to 2-5 L/min. 6. Set/Adjust temperature on thermostat 011-XX to 20oC. 7. Keep the temperature of reaction mixture in range: +20… +30oC. 8. Addition is done - one portion. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: Rinse the canister from previous op too. | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 6 | Load required amount of Triethylamine into Reactor 002-XX: 1. Weigh the required amount of Triethylamine using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.1 DETO". 3. Using the peristaltic pump, load Triethylamine via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust inert gas flow rate to 1-2 L/min. 6. Set/Adjust temperature on thermostat 011-XX to 20oC. 7. Keep the temperature of reaction mixture in range: +20… +30oC [WARNING, PARAMETER IS PC]. 8. Addition is done - pump with perist pump. Portionwise. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: no reaction at this step. | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 7 | Load required amount of DCM into Reactor 002-XX: 1. Remove the secondary package and carefully open the primary package. 2. Weigh the required amount of DCM using balance 007-XX into a jug "TP.1 DETO". 3. Pour DCM into the Reactor via handhole using a plastic funnel "TP.1 DETO". 4. Set/Adjust stirring rate in reactor 002-XX to 150-200rpm. 5. Set/Adjust inert gas flow rate to 1-2 L/min. 6. Set/Adjust temperature on thermostat 011-XX to 20oC. 7. Keep the temperature of reaction mixture in range: 15-25oC. 8. Addition is done - in one portion. 9. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; Actual temperature set : ........ oC; |
| 8 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 20oC. 2. Keep the temperature of reaction mixture in range: 20-35oC. 3. Adjust stirring rate to 150-200rpm. 4. Set/Adjust inert gas flow rate to 1-2 L/min. 5. Hold for duration: 15-25min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 9 | Start a heating/cooling operation: 1. Check that cooling water for heating/cooling circulator 011-XX is opened. 2. Set/Adjust temperature on thermostat 011-XX to 20-35oC. 3. Keep the temperature of reaction mixture in range: +20… +30oC; [WARNING, PARAMETER IS PC]. 4. Set/Adjust stirring rate to 150-200rpm. 5. Adjust inert gas flow rate to 2-5 L/min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 10 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 28oC. 2. Keep the temperature of reaction mixture in range: +20… +30oC; [WARNING, PARAMETER IS PC]. 3. Adjust stirring rate to 150-200. 4. Set/Adjust inert gas flow rate to 1-2 L/min. 5. Hold for duration: 12-44h. 6. Optional: Possible pause - keep overnight. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; Actual gas flow set: ........ L/min; |
| 11 | In-process control: In-process control (determination of IP.X conversion after 12-18h): 1. Take approximately 1mL of reaction; 2. Close the lid and create argon flow: 0.5-1 L/min; 3. Label the sample with an IPC number. For example: TBD-0XXX-Y IP.5 RM IPC 1, where Y is the batch number (copied from the header) and X is the IPC serial number. 4. Submit the sample to QC for analysis. 5. Keep stirring the reaction mixture until the IPC result is available. Decision criteria based on IPC result: • If IP.X conversion is conv. NLT95% [WARNING, PARAMETER IS CY], proceed with the workup (Operation XX). • On IPC failure: more time, 4-6h more, in extreme case add more dmsa-cl (10% from initial loading).. IPC 1: IPC no: TBD-0XXX-Y IP.X RM IPC 1 Results: Corresponds to RelS requirements:  - Yes  - No IP.X conversion: ..... % COMMENT INFO: in pre-val check ipc more often (like every 6h take a sample) | Start: __:__ End: __:__ | Result: ..............; |
| 12 | Load required amount of water into Reactor 002-XX: 1. Weigh the required amount of water using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.1 DETO". 3. Using the peristaltic pump, load water via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200. 5. Set/Adjust temperature on thermostat 011-XX to 10oC. 6. Keep the temperature of reaction mixture in range: +10… +30oC;. 7. Addition is done - temp. controlled, pump with perist pump with portions. 8. Keep/Provide temperature of loaded material in range: 15-25oC (not controlled). Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: some exothermic effect | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 13 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 20oC;. 2. Keep the temperature of reaction mixture in range: +10… +30oC;. 3. Adjust stirring rate to 150-200. 4. Hold for duration: 30-40min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 14 | Perform an extraction/separation: 1. Set/Adjust stirring in reactor to OFF. 2. Stop stirring and let layers completely separate (visual check). Expected time of separation 10-15min. 3. Perform phase separation. The bottom "description" layer is stored into container "description", top "description" layer is stored in container "description"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) COMMENT INFO: product goes into dcm. | Start: __:__ End: __:__ | Actual stirring set: ........ rpm; Actual separation time: ........ min; |
| 15 | Load required amount of water into Reactor 002-XX: 1. Weigh the required amount of water using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.1 DETO". 3. Using the peristaltic pump, load water via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200. 5. Set/Adjust temperature on thermostat 011-XX to 20oC. 6. Keep the temperature of reaction mixture in range: +10… +30oC;. 7. Addition is done - one portion. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 16 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 20oC. 2. Keep the temperature of reaction mixture in range: +10… +30oC;. 3. Adjust stirring rate to 150-200. 4. Hold for duration: 5-10min. Record required parameters into table XX. COMMENT INFO: consider separation water from the top of reactor. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 17 | Perform an extraction/separation: 1. Set/Adjust stirring in reactor to OFF. 2. Mix layers for al long as needed. 3. Perform phase separation. The bottom "description" layer is stored into container "description", top "description" layer is stored in container "description"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) | Start: __:__ End: __:__ | Actual stirring set: ........ rpm; |
| 18 | Load required amount of water into Reactor 002-XX: 1. Weigh the required amount of water using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.1 DETO". 3. Using the peristaltic pump, load water via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200. 5. Set/Adjust temperature on thermostat 011-XX to 20oC. 6. Keep the temperature of reaction mixture in range: +10… +30oC;. 7. Addition is done - one portion. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 19 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 20oC. 2. Keep the temperature of reaction mixture in range: +10… +30oC;. 3. Adjust stirring rate to 150-200. 4. Hold for duration: 5-10min. Record required parameters into table XX. COMMENT INFO: consider separation water from the top of reactor. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 20 | Perform an extraction/separation: 1. Set/Adjust stirring in reactor to OFF. 2. Mix layers for al long as needed. 3. Perform phase separation. The bottom "description" layer is stored into container "description", top "description" layer is stored in container "description"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) | Start: __:__ End: __:__ | Actual stirring set: ........ rpm; |
| 21 | Load required amount of water into Reactor 002-XX: 1. Weigh the required amount of water using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.1 DETO". 3. Using the peristaltic pump, load water via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200. 5. Set/Adjust temperature on thermostat 011-XX to 20oC. 6. Keep the temperature of reaction mixture in range: +10… +30oC;. 7. Addition is done - one portion. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 22 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 20oC. 2. Keep the temperature of reaction mixture in range: +10… +30oC;. 3. Adjust stirring rate to 150-200. 4. Hold for duration: 5-10min. Record required parameters into table XX. COMMENT INFO: consider separation water from the top of reactor. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 23 | Perform an extraction/separation: 1. Set/Adjust stirring in reactor to OFF. 2. Mix layers for al long as needed. 3. Perform phase separation. The bottom "description" layer is stored into container "description", top "description" layer is stored in container "description"; At the end layers clearly separated: Yes  / No  (in case of No, inform PM) | Start: __:__ End: __:__ | Actual stirring set: ........ rpm; |
| 24 | Start evaporation from Reactor 002-XX: 1. Set/Adjust temperature on thermostat 011-XX to 60oC with target 20-35oC. 2. Connect the membrane pump via a cold trap and turn it ON. 3. Gradually reduce the pressure within the range 300-400Torr. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 4. Maintain stirring at 150-200rpm. 5. Continue until: remove NLT40-45% of theoretical DCM. Record required parameters into table XX. COMMENT INFO: in 5.9kg took 2h | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual pressure: ........ Torr; Actual stirring set: ........ rpm; |
| 25 | Load required amount of n-Heptane into Reactor 002-XX: 1. Weigh the required amount of n-Heptane using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "TP.1 DETO". 3. Using the peristaltic pump, load n-Heptane via the loading port. 4. Set/Adjust stirring rate in reactor 002-XX to 150-200. 5. Set/Adjust temperature on thermostat 011-XX to 40oC. 6. Keep the temperature of reaction mixture in range: 20-55oC. 7. Addition is done - one portion, pulling with vacuum. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: in 3 portions with 2-3min interval. Record observations | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg Actual stirring set: ........ rpm; Actual temperature set : ........ oC; |
| 26 | Start evaporation from Reactor 002-XX: 1. Set/Adjust temperature on thermostat 011-XX to 60oC with target 20-35oC. 2. Connect the membrane pump via a cold trap and turn it ON. 3. Gradually reduce the pressure within the range 300-400Torr. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 4. Maintain stirring at 150-200rpm. 5. Continue until: remove NLT20-25% of theoretical DCM. Record required parameters into table XX. COMMENT INFO: in 5.9kg took 2h | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual pressure: ........ Torr; Actual stirring set: ........ rpm; |
| 27 | Cooling down the reaction mixture to required temp. | Start: __:__ End: __:__ |  |
| 28 | Start a hold time: 1. Set/Adjust temperature on thermostat 011-XX to 0-4oC. 2. Keep the temperature of reaction mixture in range: 0 … +5oC. 3. Adjust stirring rate to 100rpm. 4. Hold for duration: NLT 30min. 5. Optional: Possible pause - keep overnight. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual temperature set : ........ oC; Actual temperature: ........ oC; Actual stirring set: ........ rpm; |
| 29 | Filter 046-XX setup: 1. Place a PTFE seal on filtrate receiver top edge. 2. Connect suspension receiver to filtrate receiver 3. Install filter cloth into filter and fix it with the plastic ring. 4. Connect hose "TP.1 DETO" to filtrate receiver vacuum valve. Connect other end of hose to a membrane pump 001-XX. Membrane pump 001-XX connected to receiver vacuum valve:  Filtration cloth is cut and installed properly:  | Start: __:__ End: __:__ |  |
| 30 | Filter the material: 1. Set the vacuum pump to 250-350Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: until Filter cake is dry.. 4. Use a spoon "TP.1 DETO" to press and remove cracks that might appear on filter cake. COMMENT INFO: sefar cloth. Use support | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 31 | Wash filter cake: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use required amount of n-Heptane for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: should be very effeciently stirring on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: this can be used to rinse reactor walls and shaft | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 32 | Filter the material: 1. Set the vacuum pump to 250-350Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: until Filter cake is dry.. 4. Use a spoon "TP.1 DETO" to press and remove cracks that might appear on filter cake. COMMENT INFO: sefar cloth. Use support | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 33 | Wash filter cake: 1. Stop membrane pump 001-XX. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Use required amount of n-Heptane for washing Filter cake 3. Ensure loaded material temperature is 15-25oC (not controlled). 4. Mix on filter: should be very effeciently stirring on filter. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (range: … - … kg) COMMENT INFO: this can be used to rinse reactor walls and shaft | Start: __:__ End: __:__ | Warehouse code: ............ (e.g. XXXX-XX) 1 ......... kg 2 ......... kg 3 ......... kg 4 ......... kg Total ......... kg |
| 34 | Filter the material: 1. Set the vacuum pump to 250-350Torr and start it. 2. Transfer the reaction mixture from the Reactor 001-XX to the Nutsch filter in portions, allowing filtration to proceed. 3. Continue until: until Filter cake is dry.. 4. Use a spoon "TP.1 DETO" to press and remove cracks that might appear on filter cake. COMMENT INFO: sefar cloth. Use support | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 35 | Dry the filter cake on filter under vacuum: 1. Once Filter cake is visually dry, maintain it on the filter under vacuum. 2. Continue to apply vacuum 250-350Torr for 30-50min to remove residual solvents. 3. Use a spoon "TP.1 DETO" to press and remove cracks that might appear on filter cake. 4. Consider air, moisture, and light sensitivity of the material. 5. After finishing drying on filter, pump the filtrate from the filter into waste canister and weigh it. m tara = ....... kg ; m gross weight = ....... kg ; m net "type of waste" waste = ....... kg ; | Start: __:__ End: __:__ | Actual set: ........ Torr; |
| 36 | Dry the product in oven 012-XX: 1. Check and record the tare mass of each tray into table XX.1. 2. Transfer the product (filter cake) onto trays with a layer thickness of NMT 4cm. Divide all wet product approximately evenly between all trays. Crush big lumps with shovel. 3. Place the trays in the oven. 4. Set temperature of the oven to 40-50oC with overheating protection: 60oC. 5. Adjust fan to: set 70%. 6. Set flap to set 70%. 7. Dry the product, checking the weight periodically (NLT 12h after start of drying) to determine dryness 24-36h. 8. Perform Mix/delump: Yes, after at least 12h of starting of drying. Record weights in Table X. Table X filled:  Overheating set: .......°C Actual pressure fan set: .......%; Actual flap set: .......%; Temperature after at least 1h from the start: .......°C COMMENT INFO: lead pipe into ventilation | Start: __:__ End: __:__ | Actual temperature set : ........ oC; |
| 37 | Finish drying: 1. Save heating/cooling circulator 011-XX temperature logs on the USB stick that is inside the control panel or in the USB ports. 2. Turn off membrane pump and repressurize the oven. Turn off the oven. 3. Remove the trays from the oven and weigh them. Calculate loss on drying and fill Table X. 4. Calculate process yield: | Start: __:__ End: __:__ |  |
| 38 | Pack the product: 1. Use analytical sample amount: 2g. 2. Retain sample amount: 4g. 3. Pack using: double PE bag.. 4. Consider air, moisture, and light sensitivity of the material: NO. | Start: __:__ End: __:__ |  |
| 39 | Perform the analysis according to QDQS: 1. If analysis result is YES, proceed accordingly. 2. If analysis result is NO, repeat necessary steps. COMMENT INFO: continue | Start: __:__ End: __:__ |  |
| 40 | Clean the equipment: 1. Follow the solvent sequence: MeOH, Water, Acetone. 2. Estimated time required: 3-5h. | Start: __:__ End: __:__ |  |
| 41 | Perform waste treatment: | Start: __:__ End: __:__ |  |