| Description | Times | Actual Data |
| --- | --- | --- |
| Prepare the reactor: | Start: __:__ End: __:__ |  |
| Load required amount of PW water into Reactor: 1. Weigh the required amount of PW water using balance 007-XX. 2. Connect peristaltic pump 001-XX and hose "PSDSA". 3. Using the peristaltic pump, load PW water via the loading port. 4. Addition is done - in one portion. 5. Keep/Porvide temeprature of loaded material in range: 15-25oC. Record required parameters into table XX. Specified loading: ….. kg (range: … - … kg) | Start: __:__ End: __:__ | Actual Loaded material temp.: .........; Actual Stirring rate: .........; Actual Addition rate: .........; |
| Start a heating/cooling operation: 1. Open cooling water for heating/cooling circulator 011-XX 2. Set temperature on thermostat 011-XX to 120->95oC. 3. Keep the temperature of reaction mixture in range: 90oC. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual Target Temp.: .........; Actual temperature rate: .........; Actual Stirring rate: .........; |
| Add the specified amount of hot PW into the plastic pot (from 150L reactor). 1st HCl treatment. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Addition rate: .........; |
| Add 1st portion slowly of HCl into the plastic pot. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Addition rate: .........; Actual Time: .........; |
| Add starting material PSDS-Na into the plastic pot. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Addition rate: .........; Actual Time: .........; |
| Start a hold time operation: 1. Keep the temperature of reaction mixture in range: tbd. 2. Hold for duration: NLT 30 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Target Temp.: .........; Actual Stirring rate: .........; Actual Time: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 10 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Using a peristaltic pump and a hose with filter tip, carefully transfer the acid/water mixture into the Amberlite Waste holding container. | Start: __:__ End: __:__ | Actual Exp. time: .........; Actual break point: .........; |
| Add the specified amount of hot PW from 150L reactor into the plastic pot. 2nd HCl treatment. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Addition rate: .........; |
| Add 1st portion slowly of HCl into the plastic pot. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Addition rate: .........; Actual Time: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 30 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 10 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Using a peristaltic pump and a hose with filter tip, carefully transfer the acid/water mixture into the Amberlite Waste holding container. | Start: __:__ End: __:__ | Actual Exp. time: .........; Actual break point: .........; |
| Add the specified amount of hot PW from 150L reactor into the plastic pot. 1st wash. | Start: __:__ End: __:__ | Actual Loaded material temp.: .........; Actual Stirring rate: .........; Actual Addition rate: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 15 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 10 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Using a peristaltic pump and a hose with filter tip, carefully transfer the acid/water mixture into the Amberlite Waste holding container. | Start: __:__ End: __:__ | Actual Exp. time: .........; Actual pH check: .........; |
| Add the specified amount of PW into the reactor for heating. 2nd refill of 150L reactor. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Addition rate: .........; |
| Start a heating/cooling operation: 1. Open cooling water for heating/cooling circulator 011-XX 2. Set temperature on thermostat 011-XX to 120->95oC. 3. Keep the temperature of reaction mixture in range: 90oC. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual Target Temp.: .........; Actual temperature rate: .........; Actual Stirring rate: .........; |
| Add the specified amount of hot PW from 150L reactor into the plastic pot. 2nd wash. | Start: __:__ End: __:__ | Actual Loaded material temp.: .........; Actual Stirring rate: .........; Actual Addition rate: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 15 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 10 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Using a peristaltic pump and a hose with filter tip, carefully transfer the acid/water mixture into the Amberlite Waste holding container. | Start: __:__ End: __:__ | Actual Exp. time: .........; Actual pH check: .........; |
| Add the specified amount of hot PW from 150L reactor into the plastic pot. 3rd wash. | Start: __:__ End: __:__ | Actual Loaded material temp.: .........; Actual Stirring rate: .........; Actual Addition rate: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 15 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 10 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Using a peristaltic pump and a hose with filter tip, carefully transfer the acid/water mixture into the Amberlite Waste holding container. | Start: __:__ End: __:__ | Actual Exp. time: .........; Actual pH check: .........; |
| Add the specified amount of PW into the reactor for heating. 3rd refill of 150L reactor. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Addition rate: .........; |
| Start a heating/cooling operation: 1. Open cooling water for heating/cooling circulator 011-XX 2. Set temperature on thermostat 011-XX to 120->95oC. 3. Keep the temperature of reaction mixture in range: 90oC. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual Target Temp.: .........; Actual temperature rate: .........; Actual Stirring rate: .........; |
| Add the specified amount of hot PW from 150L reactor into the plastic pot. 4th wash. | Start: __:__ End: __:__ | Actual Loaded material temp.: .........; Actual Stirring rate: .........; Actual Addition rate: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 15 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Start a hold time operation: 1. Hold for duration: NLT 10 min. Record required parameters into table XX. | Start: __:__ End: __:__ | Actual Stirring rate: .........; Actual Time: .........; |
| Using a peristaltic pump and a hose with filter tip, carefully transfer the acid/water mixture into the Amberlite Waste holding container. | Start: __:__ End: __:__ | Actual Exp. time: .........; Actual pH check: .........; |
| In-process control: 1. Use IPC Method: pH and turbidity. 2. Expected Result: pH NLT 4, turbidity pass. 3. On IPC failure: Repeat water wash with 80 L hot PW until IPC passes. 4. On IPC pass: Continue next op. 5. Estimated time for analysis: 30 m. | Start: __:__ End: __:__ | Actual IPC Method: .........; Actual Sample amount: .........; Actual Expected Result: .........; Actual IPC Failure Action: .........; Actual IPC Pass action: .........; Actual Estim. Time of analysis: .........; |
| Dry the product in oven 012-XX: 1. Set temperature on thermostat 011-XX to 75-85 oC with overheating protection: ???. 2. Adjust fan setting within range: 0.5. 3. Set flap to 0.5. 4. Ensure layer thickness is 1-1.5 cm. 5. Mix/delump as necessary: Yes, after NLT 12h. 6. Dry for 42-48 h. | Start: __:__ End: __:__ | Actual Set temp.: .........; Actual overheating prot.: .........; Actual fan set. Range: .........; Actual flap set.: .........; Actual layer thickness: .........; Actual mixing/delumping: .........; Actual Time: .........; |
| In-process control: 1. Use IPC Method: VKF. 2. Expected Result: NMT 10.0%. 3. On IPC failure: Dry for additional 18-24h. 4. On IPC pass: Continue next op. 5. Estimated time for analysis: 30 m. | Start: __:__ End: __:__ | Actual IPC Method: .........; Actual Sample amount: .........; Actual Expected Result: .........; Actual IPC Failure Action: .........; Actual IPC Pass action: .........; Actual Estim. Time of analysis: .........; |
| Perform a sieving operation: 1. Use a sieve with size 250 um. 2. Process portion size: ???. 3. Consider air, moisture, and light sensitivity of the material. | Start: __:__ End: __:__ | Actual Sieve size: .........; Actual portion size: .........; Actual Time: .........; Actual air, moisture, light sensitivity of the material: .........; |
| Clean the equipment: 1. Follow the solvent sequence: water, hot gun?. 2. Estimated time required: 1-2h. | Start: __:__ End: __:__ | Actual solvent sequence: .........; Actual estim. time required: .........; |
| Inspect milling balls before use. Discard any milling balls that are found out-of-round, cracked, or showing any sign of deterioration | Start: __:__ End: __:__ | Actual number of jars: .........; Actual bead count per jar: .........; |
| Perform a milling operation: | Start: __:__ End: __:__ | Actual portion size J1: .........; Actual portion size J2: .........; Actual portion size J3: .........; |
| Perform a milling operation: 1. Mill for 96.5 hours. | Start: __:__ End: __:__ | Actual time: .........; Actual counts J1: .........; Actual counts J2: .........; Actual counts J3: .........; |
| In-process control: 1. Use IPC Method: PSD by Laser Diffration. 2. Expected Result: 6 - 10 um. 3. On IPC failure: mill for additional 8h. 4. On IPC pass: pack sample. 5. Estimated time for analysis: 2h. | Start: __:__ End: __:__ | Actual IPC Method: .........; Actual Sample amount: .........; Actual Expected Result: .........; Actual IPC Failure Action: .........; Actual IPC Pass action: .........; Actual Estim. Time of analysis: .........; |
| Pack the product: 1. Pack using: double PE-bag in poly container. | Start: __:__ End: __:__ | Actual Sieve size: .........; Actual packing material: .........; |
| Clean the equipment: | Start: __:__ End: __:__ |  |
| Perform waste treatment: | Start: __:__ End: __:__ |  |
| Perform the analysis according to QDQS: 1. If analysis result is YES, proceed accordingly. 2. If analysis result is NO, repeat necessary steps. | Start: __:__ End: __:__ | Actual YES: .........; Actual NO: .........; |