| Operation | Description |
| --- | --- |
| loading-Solid | Load [name] into reactor 002-XX: 1. Weigh [name] on balance 007-XX into jug "[project/TP code]". 2. Load [name] into reactor via handhole using a plastic scoop and plastic funnel "[project/TP code]". 3. Set/Adjust stirring rate in reactor to [Stirring] (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to [Inert gas flow rate]. 5. Adjust pH to: [pH check]. Measurement is done after 3-5 min from last addition. 6. Set/Adjust temperature on thermostat 011-XX to [Set temp] (keep jacket/RM temp. difference below 50 °C). 7. Keep reaction mixture temperature in range: [Target Temp.]. 8. Addition speed – [Addition rate]. 9. Loading is performed during: [Time]. 10. Keep/Provide temperature of loaded material in range: [Loaded material temp.]. 11. Optional: Possible pause – [break point]. Fill Table X at least every Y minutes. Specified loading: ….. kg (… – … kg) |
| loading-liquid <5L | Load [name] into reactor 002-XX: 1. Weigh [name] on balance 007-XX into jug "[project/TP code]". 2. Pour [name] into reactor via handhole using a plastic funnel "[project/TP code]". 3. Set/Adjust stirring rate in reactor to [Stirring] (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to [Inert gas flow rate]. 5. Adjust pH to: [pH check]. Measurement is done after 3-5 min from last addition. 6. Set/Adjust temperature on thermostat 011-XX to [Set temp] (keep jacket/RM temp. difference below 50 °C). 7. Keep reaction mixture temperature in range: [Target Temp.]. 8. Addition speed – [Addition rate]. 9. Loading is performed during: [Time]. 10. Keep/Provide temperature of loaded material in range: [Loaded material temp.]. 11. Optional: Possible pause – [break point]. Fill Table X at least every Y minutes. Specified loading: ….. kg (… – … kg) |
| loading-liquid >5L | Load [name] into reactor 002-XX: 1. Weigh [name] on balance 007-XX. 2. Transfer [name] into reactor with peristaltic pump and hose "[project/TP code]" via handhole (fix hose to the hole with plastic ties). Set peristaltic pump to [Perist. Pump set]. 3. Set/Adjust stirring rate in reactor to [Stirring] (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to [Inert gas flow rate]. 5. Adjust pH to: [pH check]. Measurement is done after 3-5 min from last addition. 6. Set/Adjust temperature on thermostat 011-XX to [Set temp] (keep jacket/RM temp. difference below 50 °C). 7. Keep reaction mixture temperature in range: [Target Temp.]. 8. Addition speed – [Addition rate]. 9. Loading is performed during: [Time]. 10. Keep/Provide temperature of loaded material in range: [Loaded material temp.]. 11. Optional: Possible pause – [break point]. Fill Table X at least every Y minutes. Specified loading: ….. kg (… – … kg) |
| loading-dropwise addition | Load [name] into Reactor 002-XX dropping funnel: 1. Weigh [name] on balance 007-XX into jug "[project/TP code]". 2. Load [name] into the dropping funnel using a plastic funnel "[project/TP code]". 3. Set/Adjust stirring rate in reactor to [Stirring] (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to [Inert gas flow rate]. 5. Adjust pH to: [pH check]. Measurement is done after 3-5 min from last addition. 6. Set/Adjust temperature on thermostat 011-XX to [Set temp] (keep jacket/RM temp. difference below 50 °C). 7. Keep reaction mixture temperature in range: [Target Temp.]. 8. Addition speed – [Addition rate]. 9. Loading is performed during: [Time]. 10. Keep/Provide temperature of loaded material in range: [Loaded material temp.]. 11. Optional: Possible pause – [break point]. Fill Table X at least every Y minutes. Specified loading: ….. kg (… – … kg) |
| loading-suspend | Prepare suspension of [name1] in [name2] and load into Reactor 002-XX: 1. Weigh Materials: a. Weigh the [name1] using Balance 007-XX. b. Weigh the [name2] using Balance 007-XX. 2. Prepare Suspension: a. In a suitable container (5L jug), add the [name2]. b. Slowly add [name1] into the [name2] while stirring to create a suspension. c. Ensure that the suspension is well mixed. d. Note: [name1] is a dusty product; wear appropriate personal protective equipment (PPE) including masks and protective clothing during handling. 3. Load Suspension into Reactor: a. Set/Adjust temperature on thermostat 011-XX to [Set temp] (keep jacket/RM temp. difference below 50 °C). b. Keep the temperature of reaction mixture in range: [Target Temp]. c. Set/Adjust inert gas flow rate to [Inert gas flow rate]. d. Load the suspension into Reactor by carefully pouring through a funnel "[project/TP code]" trying to minimize splashing and exposure. 4. Stirring: a. Set the stirring speed to [Stirring] (recommended range) (range is recommended and can be adjusted). b. Note: Ensure stirring is adequate to keep the suspension homogeneous. Record required parameters into table XX every YY min. Specified loading [name1]: ….. kg (… – … kg) Specified loading [name2]: ….. kg (… – … kg) Warehouse code of [name1]: ........................ . (eg. XXXX-XXX) Loading of [name1]: Warehouse code of [name2]: ........................ . (eg. XXXX-XXX) Loading of [name2]: |
| reactor prep. | Reactor 002-XX preparation: 1. Heat Reactor to [Heating to the temp.]. 2. Hold Reactor at temperature for [hold time at temp]. 3. Set Argon flow during heating to [Argon flow during heating]. 4. Cool Reactor to [cooling to temp.]. 5. Set Argon flow during cooling to [Argon flow during cooling]. |
| packing | Pack the product: 1. Use analytical sample amount: [analytical sample amount]. 2. Retain sample amount: [ret. Sample amount.]. 3. Pack using: [packing material]. 4. Consider air, moisture, and light sensitivity of the material: [air, moisture, light sensitivity of the material]. |
| Analyzing, correspond to QDQS | Perform the analysis according to QDQS: 1. If analysis result is YES, proceed accordingly. 2. If analysis result is NO, repeat necessary steps. |
| eq. cleaning | Clean the equipment: 1. Follow the solvent sequence: [solvent sequence]. 2. Estimated time required: [estim. time required]. |
| waste treatment | Perform waste treatment: 1. Process water waste: [water waste]. 2. Process organic waste: [organic waste]. 3. Process solid waste: [solid waste]. |
| unloading | Perform an unloading: 1. Unload [name] using appropriate method: [method of transf.]. 2. Measure unloaded amount: [Amount] kg. 3. Maintain stirring rate during unloading at [Stirring] (recommended range) (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to [Inert gas flow rate]. 5. Optional: Possible pause – [break point]. |
| decant | Decant liquid part from reactor 002-XX: 1. Set/Adjust thermostat 011-XX to [Set temp]. 2. Keep reaction mixture temperature in range: [Target Temp]. 3. Turn off stirrer. Let precipitate settle on the bottom of reactor for [Time]. 4. Set/Adjust inert gas flow rate to [Inert gas flow rate]. 5. Optional: Possible pause – [break point]. 6. After required time passed, pump liquid part (top layer) from reactor into “direction”. Use peristaltic pump and hose "[project/TP code]". Peristaltic pump set: [Perist. Pump set]. 7. Stop when [End point]. |
| heating/cooling | Start heating/cooling with thermostat 011-XX: 1. Check that cooling water for 011-XX is opened. 2. Set/Adjust temperature on 011-XX to [Set temp] (keep jacket/RM temp. difference below 50 °C). 3. Keep reaction mixture temperature in range: [Target Temp]. 4. Provide temperature gradient: [cooling/heating grad.]. 5. Set/Adjust stirring in reactor 002-XX to [Stirring] (range is recommended and can be adjusted). 6. Set/Adjust inert gas flow rate to [Inert gas flow rate]. Fill Table X at least every Y minutes. |
| hold time | Hold time: 1. Set/Adjust thermostat 011-XX to [Set temp]. 2. Keep reaction mixture temperature in range: [Target Temp] for [Time]. 3. Set/Adjust stirring in reactor 002-XX to [Stirring] (range is recommended and can be adjusted). 4. Set/Adjust inert gas flow rate to [Inert gas flow rate]. 5. Optional: Possible pause – [break point]. Fill Table X at least every Y minutes. |
| Filter prep. | Filter 046-6-7 setup: 1. Place a PTFE seal on filtrate receiver top edge. 2. Connect suspension receiver to filtrate receiver. 3. Install filter cloth into filter and fix it with the plastic ring. 4. Connect hose "[project/TP code]" to filtrate receiver vacuum valve. Connect other end of hose to a membrane pump. Membrane pump connected to receiver vacuum valve: Filtration cloth is cut and installed properly: Filter 046-2-13 setup: 1. Connect suspension receiver to glass receiver from reactor 002-XX with norprene hose "[project/TP code]". 2. Connect reactor receiver to membrane pump with norprene hose "vacuum". 3. Install filter cloth into filter and fix it with the plastic ring. Filter is connected to Reactor's receiver vessel: Membrane pump is connected to Reactor's receiver vacuum valve: Filtration cloth is cut and installed properly: |
| IPC | In-process control: In-process control (determination of IP.X conversion after [Sample taken in]): 1. Take approximately 1 mL of reaction mixture into a 20 mL vial; 2. Sample is quenched: [Sample quenching]. 3. Purge the vial with argon; 4. Label the sample with an IPC number. 5. Submit the sample to QC for analysis. 6. Keep stirring the reaction mixture until the IPC result is available. 7. Estimated time for analysis: [Estim. Time of analysis] Acceptance criteria based on IPC result: • If IP.X conversion is [Expected Result], proceed with the workup (Operation XX). • On IPC failure: [IPC Failure Action]. • On IPC pass: [IPC Pass action]. IPC 1: TBD-0XXX-.... IP.X RM IPC 1 Results: Corresponds to RelS requirements: – Yes / – No IP.X conversion: ….. % |
| solution prep. | Prepare "description" solution according to table 1.X: Solution is prepared: |
| evap. | Start evaporation in reactor 002-XX: 1. Set/Adjust temperature thermostat 011-XX to [Set temp] with target [Target Temp] (keep jacket/RM temp. difference below 50 °C). 2. Connect a membrane pump via a cold trap and turn it ON. 3. Gradually reduce pressure [Pressure set range]. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 4. Set/Adjust stirring to [Stirring] (range is recommended and can be adjusted). 5. Continue until: [End point]. 6. Set/Adjust inert gas flow rate to [Inert gas flow rate]. 7. Optional: Possible pause – [break point]. Fill Table X at least every Y minutes. |
| evap. Rota | Start evaporation in rotary evaporator 009-XX: 1. Check 009-XX bath level. Add more RO water if needed. 2. Connect and tighten flask "[project/TP code]" using a special key. 3. Set/Adjust bath temperature to [Set temp]. 4. Turn ON condenser cooling water. 5. Connect membrane pump via a cold trap and turn it ON. 6. Gradually reduce the pressure within the range [Pressure set range]. Make sure the condenser is not overflooded with condensed solvent, adjust pressure accordingly. 7. Set/Adjust stirring to [Stirring] (range is recommended and can be adjusted). 8. Connect hose "[project/TP code]" for loading solution, open loading valve and load solution for evaporation. 9. Continue until: [End point]. 10. Set/Adjust inert gas flow rate to [Inert gas flow rate]. 11. Optional: Possible pause – [break point]. Fill Table X at least every Y minutes. |
| extr./separ. | Perform extraction/separation: 1. Set/Adjust stirring in reactor to [Stirring] (recommended range) (range is recommended and can be adjusted). 2. Mix layers for [Time]. 3. Stop stirring and let layers completely separate (visual check). Expected time of separation [Exp. time]. 4. Perform phase separation. Bottom "description" phase is stored into canister "[project/TP code]", top "description" layer is stored in canister "[project/TP code]". At the end layers clearly separated: Yes / No (in case of No, inform PM) |
| filtration | Filter the material: 1. Keep stirring in Reactor 002-XX to 150–60 rpm (gradually decreasing). 2. Connect norprene hose "[project/TP code]" from reactor valve and through peristaltic pump to Nutsch filter. Set the peristaltic pump to 40–60 %. 3. Set the vacuum pump to [Pressure set] and start it. 4. Start peristaltic pump, open reactor’s valve and transfer the reaction mixture from the Reactor to the Nutsch filter in portions, allowing filtration to proceed. 5. Record actual achieved pressure during filtration: [actual pressure]. 6. Continue until: [End point]. 7. Use a spoon "[project/TP code]" to press and remove cracks that might appear on filter cake. NB! Once Reactor’s receiver is 2/3 full, stop the process, release the vacuum from the system, connect filter outlet to atmosphere and empty filtrate into "[project/TP code]". Resume the process afterwards. |
| Candle Filter prep. | Candle filter preparation: 1. Assemble the filter using the [filter size] cartridge and filter case. 2. Secure the case with the special key. 3. Connect Norprene hoses: attach the inlet to the larger pore side and the outlet to the smaller pore side. |
| filtration with candle filter | Filter the reaction mass using candle filter: 1. Set flow rate to [Flow rate]. 2. Continue until reaching end point: [End point]. |
| Washing Filter cake | Wash filter cake with [name]: 1. Stop membrane pump. Connect outlet to the atmosphere (to prevent filter cloth from floating). 2. Weigh [name] for washing Filter cake using balance 007-XX. Then pour onto filter cake. 3. Ensure loaded material temperature is [Loaded material temp.]. 4. Mix on filter: [Mixing on filter]. 5. Restart membrane pump and continue filtration. Specified loading: ….. kg (… – … kg) |
| Dry on filter | Dry the filter cake on filter under vacuum: 1. Once Filter cake is visually dry, maintain it on the filter under vacuum. 2. Continue to apply vacuum [Pressure set] for [Time] to remove residual solvents. 3. Use a spoon "[project/TP code]" to press and remove cracks that might appear on filter cake. 4. Consider air, moisture, and light sensitivity of the material. 5. After finishing drying on filter, pump the filtrate from the filter into waste canister and weigh it. m tara = ....... kg; m gross weight = ....... kg; m net "type of waste" waste = ....... kg |
| drying in vac. | Dry the product in vacuum oven 012-XX: 1. Check and record the tare mass of each tray into table XX.1. 2. Transfer the product (filter cake) onto trays with a layer thickness of [layer thickness]. Divide all wet product approximately evenly between all trays. Crush big lumps with shovel. 3. Place the trays in the vacuum oven. 4. Connect vacuum pump to the oven via cold trap with dry ice. 5. Set overheating protection of the oven to: [overheating prot.]. 6. Set the oven temperature to: [Set temp] and start the pump with set: [Pressure set range]. 7. Dry the product, checking the weight periodically (NLT 12 h after start of drying) to determine dryness [Time]. In 2–3 h weight loss should be less than 2–3 %. 8. Perform Mix/delump: [mixing/delumping]. Record weights in Table X. Table X filled: Overheating set: ....... °C Pressure after at least 1 h from the start: ...... Torr Temperature after at least 1 h from the start: ....... °C |
| drying at atm. | Dry the product in oven 012-XX: 1. Check and record the tare mass of each tray into table XX.1. 2. Transfer the product (filter cake) onto trays with a layer thickness of [layer thickness]. Divide all wet product approximately evenly between all trays. Crush big lumps with shovel. 3. Place the trays in the oven. 4. Set temperature of the oven to [Set temp] with overheating protection: [overheating prot.]. 5. Adjust fan to: [fan set.]. 6. Set flap to [flap set.]. 7. Dry the product, checking the weight periodically (NLT 12 h after start of drying) to determine dryness [Time]. 8. Perform Mix/delump: [mixing/delumping]. Record weights in Table X. Table X filled: Overheating set: ....... °C Fan set: ....... %; Flap set: ....... %; Temperature after at least 1 h from the start: ....... °C |
| end of drying in oven | Finish drying: 1. Save thermostat 011-XX temperature logs on the USB stick that is inside the control panel or in the USB ports. 2. Turn off membrane pump and repressurize the oven. Turn off the oven. 3. Remove the trays from the oven and weigh them. Calculate loss on drying and fill Table X. 4. Calculate process yield: |
| centrifuging | Perform a centrifuging: 1. Initiate with loading spin at [loading spin]. 2. Proceed with process spin at [process spin]. 3. Process portion size: [portion size]. |
| H2 reactor preparation | The high-pressure reactor 037-X is prepared for work according to user manual: 1. Stirrer drive 021-XX is installed. 2. Thermosensor 003-XX is connected. 3. Reactor is connected to H₂ gas cylinder (red hose). 4. Reactor is connected to argon gas cylinder (blue hose). 5. The vacuum line is connected to membrane pump or analogue. 6. Exhaust of outlet line is directed to ventilation. 7. Manometer 023-XX is installed. The reactor is set up as required: Cooling water connected: Argon is connected: Hydrogen is connected: Outlet hose end is placed under the fume hood: Membrane pump connected: 001-…… Connection to tap water is free of leaks: |
| H2 reaction | Perform a hydrogenation/pressurized reaction: 1. Set temperature on thermostat 011-XX to [Set temp] with target temperature in reaction mixture within [Target Temp]. 2. Adjust stirring to [Stirring] (recommended range) (range is recommended and can be adjusted). 3. Run the reaction for [Time]. 4. Set hydrogen pressure to [H2 pressure]. |
| sieving | Perform a sieving: 1. Use a sieve with size [Sieve size]. 2. Process portion size: [portion size]. 3. Consider air, moisture, and light sensitivity of the material. |
| milling | Perform a milling: 1. Use a mill of size [mill size]. 2. Process portion size: [portion size]. 3. Mill for [Time]. |